## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Tool Engineering-I(Cutting Tools)[5033]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between Continuous and discontinuous chips. |  |
| b) | State and explain the tool life equation considering the effect of feed & depth of cut. |  |
| c) | Explain the term – ‘tool life for maximum production’. |  |
| d) | How do the cutting fluids influence the machineability? |  |
| e) | Show the negative and positive back rake angles with the help of sketches. |  |
| f) | State the limitations of carbon tool steels. |  |
| g) | How will you select appropriate cutting fluid for a given metal cutting operation? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the terms and illustrate them with sketches – (i) Shear plane (ii) Chip thickness ratio. |  |
| b) | Draw the merchant’s force diagram and State the assumptions made therein. |  |
| c) | While machining on a shaper under orthogonal cutting conditions, the specimen length along the stroke is 100mm, and the chip length of 40mm is obtained with a tool of 15 rake angle. Determine the shear angle and chip thickness if uncut chip thickness is 1.5mm. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define tool life. Explain different concepts of tool life. |  |
| b) | Describe any three types of tool wear giving sketches. |  |
| c) | Tool lives obtained for the speeds of 25m/min and 60m/min are 100 minutes and 9 minutes respectively. Determine – - Tool life equation - Cutting speed for 2 minute life - Tool life for cutting speed of 50m/min. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the sketches of following multiple point cutting tools: (i) Broach (ii) Spot facer. |  |
| b) | Explain the following tool signature for a single point cutting tool. 10 – 20 – 7 – 6 – 8 – 15 - 0.8 mm |  |
| c) | How are cemented carbides manufactured? (Answer in brief). |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following properties of tool materials: (i) Hot hardness (ii) Toughness (iii) Wear resistance. |  |
| b) | Comment on the storage and disposal of cutting fluids. |  |
| c) | State the functions of cutting fluids. |  |

………2/-

-2-

| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Chip formation process |  |
| b) | Concepts of tool failure |  |
| c) | Form tools |  |
| d) | Selection of tool material |  |
| e) | Gaseous cutting fluids |  |

***********