## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A jet of water of diameter 35 mm moving with a velocity of 25 m/s, strikes a curved fixed symmetrical plate at the centre. Find the force exerted by the jet if it is deflected through an angle of 135 at the outlet of the curved plate. |  |
| b) | State the function of the following details of Pelton wheel: i) Bucket splitter ii) Deflector iii) Spear. |  |
| c) | Define ‘Static head’ and ‘Manometric head’ of a centrifugal pump. |  |
| d) | Why are air vessels used in reciprocating pump? |  |
| e) | Give the applications of screw pump. |  |
| f) | What is F.R.L unit? State its function in pneumatic system. |  |
| g) | Write down the symbols for the following hydraulic components: i) unloading valve ii) check valve iii) Double acting cylinder. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Pelton wheel has a mean bucket speed of 10 m/s with a jet of water flowing at the rate of 0.7 m3/s under a head of 30m. The buckets deflect the jet through an angle of 160. Calculate the power and the hydraulic efficiency of the turbine. Take Cv=0.98 and assume the buckets to be smooth. |  |
| b) | Draw a neat sketch of governing mechanism for reaction turbine and label the parts. Why is a relief valve provided? |  |
| c) | Explain hydraulic, mechanical and overall efficiency of a Pelton wheel. State the condition for maximum efficiency of Pelton wheel. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Following are the results of test conducted on a centrifugal pump: Vacuum gauge reading on the suction side = 250mm of mercury; Pressure gauge reading on the discharge side= 0.14 N/mm2; Height difference between the two gauges = 0.45 m; Rate of discharge = 85 lit/s; Diameter of suction and delivery pipe equal to 200 mm each. Find the manometric head of the pump. If overall efficiency is 75%, find the shaft power required to run the pump. |  |
| b) | Describe multistage centrifugal pump with i) Impellers in series and ii) Impellers in parallel. |  |
| c) | Explain the various types of losses occurring in centrifugal pump. Show them with the help of diagram. | ………2/- |

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A single acting reciprocating pump running at 50 r.p.m delivers 0.03 m3/s of water. The diameter of the piston is 400mm and stroke length of 300mm. The pump lifts water to a total height of 12m. Find the theoretical discharge, % slip and power required to drive the pump. |  |
| b) | Discuss the effect of friction and acceleration in suction and delivery pipe of reciprocating pump with the help of an indicator diagram. |  |
| c) | Show that the work saved against friction by fitting air vessel to the single acting reciprocating pump is 84.8%. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Kaplan turbine produces 50,000 KW when working under a head of 25m with an overall efficiency of 90%. Find the runner diameter and the specific speed of the turbine. Take speed ratio= 2.0 and flow ratio = 0.6. The hub diameter is 0.35 times the outside diameter. |  |
| b) | State the functions of oil seals used is hydraulic control system. Mention two types of oil seals with their applications. |  |
| c) | What are the different types of direction control valves used in pneumatic control system and how are they actuated? |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare and discuss the performance of hydraulic turbines with the help of operating characteristics curves. |  |
| b) | Explain the working of any one type of variable delivery pump with the help of a neat sketch. |  |
| c) | Explain the working of a sequential hydraulic circuit with the help of a circuit diagram. |  |

***********