## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./EI/COMP/EC/MED/E&EE

Subject: Digital Electronics-II(4138)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare – Series adder and parallel adder. |  |
| b) | Draw logic diagram of 1:4 Demultiplexer. Write down the truth table. |  |
| c) | Define following w.r.t. DAC: Settling time and Offset Error. |  |
| d) | Draw the block diagram of Successive Approximation method of ADC. |  |
| e) | Draw Astable multivibarator using IC 555. Write the formula for TON and TOFF. |  |
| f) | Periodic refreshing is required for DRAMs - Justify. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw diagram of 4 bit Adder/Subtractor circuit. Explain the operation with following binary operations. 1001 + 0101 and 1001 - 0101 |  |
| b) | Draw logic diagram for full Subtractor and explain operation with truth table. |  |
| c) | Perform following binary operations. x) 1111 x 1010 y) 1111  0011 |  |
| d) | Draw neat diagram of single cell of static RAM and explain the operation. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What is multiplexer? Explain nibble multiplexer with logic diagram. |  |
| b) | What is decoder driver? Explain different types of decoder driver. |  |
| c) | Compare – Decoder and Demultiplexer. |  |
| d) | Draw 2 to 4 decoder. Design half Substractor using this decoder. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a diagram for 3-bit D to A converter using Binary weighted method. What will be the output of above DAC for digital input 101(2), if logic 0 = OV & Vref = 12V . List two disadvantages of above method. |  |
| b) | Explain with neat diagram, single slope method of converting analog to digital data. |  |
| c) | i) Find the percentage resolution of 9-bit DAC which uses the ladder network. If the full scale output voltage of converter is +5V, find resolution in volts also. ii) List any three points of comparison between simultaneous method of ADC using parallel comparators and digital ramp (Counter method) method of ADC. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw monostable multivibrator using functional diagram of 555 timer. Explain the operation with waveforms. Write down the formula for output pulse after application of trigger pulse. |  |
| b) | Write a short note on contact bounce and debounce circuits. |  |
| c) | Obtain 1024 x 8 memory using 256 x 4 memory. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare PLA and PAL. |  |
| b) | Write a short note on PROMs. |  |
| c) | Explain with example, how can the capacity of PLA be expanded. |  |

***********