## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Welding Engineering(5041)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the factors affecting selection of welding process. |  |
| b) | Explain the elements used to represent weld on drawings. |  |
| c) | How are electrodes manufactured? Explain in brief. |  |
| d) | What are the advantages of Invertor over transformer as welding power source? |  |
| e) | Define Bulf welding. |  |
| f) | Explain undercutting and overlapping as welding defects. |  |
| g) | Compare Destructive and Non-destructive tests of welded specimen. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How does temperature distribution takes place in and around the weld? Explain. |  |
| b) | Explain with sketches the various types of flames in Oxy- acetylene welding. |  |
| c) | What is the purpose of pressure regulator? Differentiate between acetylene and oxygen pressure regulator. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Gas Metal Arc welding process with neat sketch. |  |
| b) | What do you understand by welding electrodes? Give its classification. |  |
| c) | Explain the techniques for minimizing distortion of welded plates. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare welding, brazing and soldering. |  |
| b) | Explain Ultrasonic welding process with neat sketch. |  |
| c) | Discuss the various accessories used for protection of welder. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the causes and remedies of inadequate joint preparation. |  |
| b) | What is the significance of Tensile test? Explain how all weld metal tensile test specimen is prepared. |  |
| c) | With a neat sketch, explain Magnetic particle testing. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Braze welding |  |
| b) | Welding joints |  |
| c) | Distortion |  |
| d) | Fire hazards |  |

***********