## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT/M

Subject: Hydraulics(4025)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (i) Mass Density (ii) Weight Density (iii) Specific gravity. |  |
| b) | What is the effect of temperature on viscosity of fluids and why? |  |
| c) | If the Barometer reads 700 mm and a Bourdon Tube pressure gauge at appoint in a flow system reads 500 KN/m2. What is the absolute pressure at that point where Bourdon Tube pressure gauge is attached? |  |
| d) | Give the statement of Bernoulli’s theorem and the assumptions made in deriving Bernoulli’s equation. |  |
| e) | Give the classification of Orifices. |  |
| f) | What is a Cipoletti weir? What is the benefit in using a Cipoletti weir over a rectangular weir? |  |
| g) | List the different types of losses occurring in a pipe flow. |  |
| h) | Compare flow through open channel with pipe flow. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the principal and working of a Bourdon Tube pressure gauge. |  |
| b) | A circular opening, 2.5m diameter in a vertical side of a tank is closed by a disk of 2.5m diameter which can rotate about a horizontal diameter. Determine (i) The force on the disc. (ii) The torque required to maintain the disc in equilibrium in vertical position when the head of water above the horizontal diameter is 3.5m. |  |
| c) | An Inverted differential manometer containing an oil of specific gravity 0.9 is connected to find the difference of pressures at the two points of a pipe containing water. If the manometer reading is 40 cms, find the difference in pressures at the two points. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A pipe 1, 450 mm in diameter branches into two pipes 2 and 3 of diameters 300mm and 200mm respectively. If the average velocity of water flowing in 450mm diameter pipe is 3m/s, find (i) The discharge through 450 mm diameter pipe (ii) Velocity in 200mm diameter pipe if the average velocity in 300 mm diameter pipe is 2.5m/s. |  |
| b) | A submarine fitted with a pitot static tube moves horizontally in sea. Its axis is 12m below the surface of water. The pitot static tube fixed infront of the submarine and along its axis is connected to the two limbs of a u tube manometer containing mercury, the reading of which is found to be 200mm. Find the speed of the submarine in Km/hr. Take specific gravity of sea water =1.025 times freshwater. |  |
| c) | A rectangular Orifice 1.5m wide and 1.2m deep is fitted in one side of a large tank. The water level on one side of the Orifice is 2m above the top edge of the Orifice, while on the other sides of the Orifice, the water level is 0.4m below its top edge. Calculate the discharge through the orifice if Cd for Orifice is 0.62. |  |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive an expression for the ratio of diameters at outlet and at Vena Contracta for a convergent divergent mouthpiece in terms of absolute pressure, head at Vena contracta, head of liquid above mouthpiece and atmospheric pressure head. |  |
| b) | A 40m long weir is divided in 12 equal bays by vertical posts, each 0.6 wide. Using Francis’s formula, calculate the discharge over the weir if the head over the crest is 1.2m and velocity of approach is 2 m/s. |  |
| c) | Determine the difference in the elevations between the water surfaces in the two tanks which are connected by a horizontal pipe of diameter 300mm and length 400m. The rate of flow of water through the pipe is 300 litres/s. Consider all losses and take the value of co-efficient of friction for pipe material f=0.008. Also draw the hydraulic gradient line and total energy line. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A main pipe divided into two parallel pipes which again forms one pipe. The length and diameter for the first parallel pipe are 2000m and 1.0m respectively, while the length and diameter of the second pipe are 2000m and 0.8m respectively. If the total flow in the main pipe is 3m3/s and the co-efficient of friction for each parallel pipe is same and equal to 0.005, find the rate of flow in each parallel pipe. |  |
| b) | State and derive the conditions under which the rectangular section of an open channel will be most economical. |  |
| c) | A triangular gutter, whose sides include an angle of 60 conveys water at an uniform depth of 250mm. If the discharge is 0.04m3/s, determine the gradient of the trough. Use Chezy’s constant C=52. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A trapezoidal channel having the side slope equal to 60 with the horizontal and laid on a slope of 1 in 750, carries a discharge of 10m3/s. Find the width at the base and depth of flow for most economical section. Take the value of Chezy’s constant C=66. |  |
| b) | Write short notes on any two: |  |
|  | i) Hydraulic Intensifier ii) Hydraulic Accumulator iii) Hydraulic Ram. |  |
| c) | Distinguish clearly between Centrifugal pump & Reciprocating pump. |  |

***********