## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Chemistry of Welding Processes(4231)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten of the following: | 10x1.5=15 |
| --- | --- | --- |
| a) | How is liquid oxygen converted into gaseous oxygen by using cold gasifier? |  |
| b) | State any three properties of Helium gas. |  |
| c) | How is gas quantity determined in a cylinder? |  |
| d) | State any three requirements of porous fillers used in acetylene cylinder. |  |
| e) | When is carburizing flame used in welding? |  |
| f) | Define specific heat flux. |  |
| g) | Why is there need for post cleaning of fluxes? |  |
| h) | What is the basic difference between welding and brazing? |  |
| i) | Define calorific value and state its units. |  |
| j) | How are welding electrodes classified on the basis of coating factor? |  |
| k) | State the sources of gases absorbed by molten weld pool. |  |
| l) | What are coated, clad and sintered metals? |  |
| Q.No.2. | Answer any three of the following: | 3 x 4 = 12 |
| a) | What is the principle of Linde’s method for the production of oxygen? State any two advantages of using liquid oxygen. |  |
| b) | Explain the significance of process of reduction in welding. |  |
| c) | State the characteristics of acidic and basic oxides. |  |
| d) | Why is Argon gas better suited for oxide cleaning during welding? |  |
| Q.No.3. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Draw a neat labelled diagram of carbide to water Acetylene generator. |  |
| b) | Explain the effects of presence of phosphine and aceton vapour as impurities in Acetylene. |  |
| c) | Write a note on explosibility of Acetylene in mixture with other gases. |  |
| d) | State any two properties each of any four fuel gases other than Acetylene. |  |
| Q.No.4. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Explain the structure and chemistry of oxy-acetylene flame. |  |
| b) | How does flame angle affects the efficiency of welding? |  |
| c) | Explain the functions and requirements of brazing fluxes. |  |
| d) | Explain in brief the three types of soldering fluxes. |  |

………2/-

-2-

| Q.No.5. | Answer any three of the following: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Discuss the advantages of using flux coated electrodes. |  |
| b) | Explain the extrusion process of manufacture of welding electrodes and state the care to be taken during their storage. |  |
| c) | Give an account of typical faults caused by absorption of gases by welds. |  |
| d) | Explain the types of Exothermic reactions due to Gas metal reactions. |  |
| Q.No.6. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Discuss any two methods of preventing gaseous contamination of molten weld metal. |  |
| b) | State any two properties and industrial applications of Copper and Aluminium. |  |
| c) | Draw the B.C.C and F.C.C. crystal lattice structures and explain the effects of temperature on crystal size. |  |
| d) | State any four points of distinction between metals and non-metals. |  |

***********