## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg.

Subject: Energy Management(5108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the benefits of energy conservation to Industries. |  |
| b) | Explain the importance of flattering of load curve. |  |
| c) | State the quality parameters of incoming power supply. |  |
| d) | List down the different methods used to determine motor loading. |  |
| e) | Define pump efficiency and state the formula for hydraulic power of pump. |  |
| f) | Briefly explain ultrasonic flow meter. |  |
| g) | Define i) Lumen ii) Colour Rendering Index. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on Energy conservation Act 2001. |  |
| b) | List down the energy strategies for future under immediate, short and long term basis. |  |
| c) | Write a short note on classification of Energy Audit. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the various components of electrical tariff system. |  |
| b) | With the help of neat sketch, explain the generation of load curve and rescheduling of loads. |  |
| c) | With the help of a neat sketch of demand curve, explain the concept of maximum demand. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the advantages of power factor improvement by capacitor addition. |  |
| b) | Briefly explain the calculation for capacitor sizing for power factor improvement. The utility bill shows an average p.f. of 0.72 with an average KW of 627. How much KVAR capacitors are required to improve the power factor to 0.95? |  |
| c) | A chemical plant has 1500 KVA transformer. The initial demand of the plant 1160 KVA with p.f. 0.7. Determine the percentage loading of transformer. How much capacitors are required to improve the p.f. to 0.89. Also estimate the reduction in KVA after p.f. improvement. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the steps used to determine the efficiency of motor. |  |
| b) | Explain the procedure for assessment of performance of lighting system. |  |
| c) | Determine the pump efficiency and overall system efficiency for the pump having following data: Pump flow= 0.4 m3/s Height of cooling = 5m Power absorbed = 325 KW Motor efficiency = 88% Suction head = +2m Type of drive = direct coupled Delivery head = 55m Density of water =996 Kg/m3. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on maximum demand controller. |  |
| b) | Write a short note on energy efficient lighting controls. |  |
| c) | Explain the concept of VFD and its power conversion. |  |

***********