## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./EC/COMP/MED/E&EE

Subject: Analog Electronics(4140)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the symbol and equivalent circuit of an op-amp. |  |
| b) | What is meant by time scaling? |  |
| c) | Draw the circuit diagram and frequency response of first order low pass Butterworth filter. |  |
| d) | Mention the disadvantages of a basic differentiator circuit. How can you eliminate it? |  |
| e) | What is frequency scaling? How is it done? |  |
| f) | Explain any one application of Phase Locked Loop. |  |
| g) | Draw the set up to solve the following equation d2ydt2+5.4dydt+0.58y=c(t). |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit diagram and explain the working of a Voltage shunt amplifier using op-amp. Obtain its gain. |  |
| b) | With a neat diagram explain Current to Voltage Converter. |  |
| c) | Explain the block diagram of an operational amplifier. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with neat diagram the operation and frequency response of practical integrator circuit. |  |
| b) | Explain working of basic comparator with positive reference voltage. Draw the circuit diagram and waveform. |  |
| c) | With the help of neat diagram and waveform, explain working of Monostable Multivibrator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat circuit diagram and waveform explain the working of Wein-bridge oscillator circuit using op-amps. |  |
| b) | Explain averaging and summing amplifier using non-inverting configuration. |  |
| c) | Explain the staircase generator using op-amps. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a low pass Butterworth filter with AF=3 and fH=1KHZ. Also frequency scale the filter for 2KHz. |  |
| b) | Explain the block diagram of Phase Locked Loop. |  |
| c) | With a neat block diagram explain the working of a switching regulator. |  |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | Diode function generator |  |
| b) | IC 565 |  |
| c) | Op-amp as peak detector |  |
| d) | Functional block diagram of 723 voltage regulator |  |

***********