## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Piping Fabrication & Installation(5342)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the specification of a pipe. |  |
| b) | Differentiated between PFD and P&ID drafts of a piping system. |  |
| c) | Sketch the different pressure tight pipe joints used in piping and their area of applications. |  |
| d) | Explain the term “Nozzle Schedule”. Explain its importance. |  |
| e) | Give the use of the following pipe fittings: i) Coupling ii) Nipple iii) Gasket. |  |
| f) | What is the piping flexibility? What is its importance? |  |
| g) | Explain in short the radiographic test of a pipe weld joint. |  |
| Q.No.2. | Design a pipe of material ASTM A106 GB to fabricate a pipeline to conduct a fluid at 325C, delivery pressure 7 Kg/cm2 and discharge of 40 liters/sec. The total length of the pipeline is 7725 m and have in its length 10 numbers of short Radius 90 elbows, 2 numbers of gate valves, 1 number Non- return valve and 2 numbers of reducers. (Use the given data) | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the Front View, Side View and Top View of the portion of the pipeline whose ‘PLAN’ is given in fig.1. (Draw the views out-off scale, but dimension carefully the views) | (8) |
| b) | Make a bill of Material of the pipeline shown in fig.1, Q.3a. Neglect the weight of the pipes and pipe fittings. | (4) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the compression pipe bending process. |  |
| b) | Determine the minimum length of a straight 60 mm O.D. pipe, required to produce a fabricated four piece pipe bend of bend angle 110 and bend radius at the centre of pipe 100mm. |  |
| c) | Explain the need of mandrels in pipe bending. Sketch the different types of mandrels used in pipe bending. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure to conduct a spark test on a plastic pipe weld joint. |  |
| b) | Why and when piping insulation is required? Explain the procedure involved to apply insulation on a pipeline. |  |
| c) | Explain the different design considerations to be taken care, while fabricating a steam pipeline. |  |

………2/-

-2-

| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Use of Valves in pipelines |  |
| b) | Corrosion tables and their importance |  |
| c) | Major phases in piping Installation |  |
| d) | Need of Pipe supports |  |
| e) | Piping material test certificates |  |

### DATA

#### Table 1:-   Pressure drop in Pipes

| PIPE NB US GPM | Pressure drop (PSI) per 100 ft long Sch 40 Pipe |  |  |
| --- | --- | --- | --- |
|  | 5 | 6 | 8 |
| 500 | 8.02 | 5.55 | 3.21 |
| 550 | 8.82 | 6.11 | 3.53 |
| 600 | 9.62 | 6.66 | 3.85 |

#### Table 2:-   Allowable stresses for ASTM A106 GB

| Temperature of fluid F | - 20 to 650 | 700 | 750 | 775 |
| --- | --- | --- | --- | --- |
| Allowable stresses (PSI) | 15,000 | 14,300 | 12,900 | 11,850 |

#### Table 3:-   Pipe properties

| Pipe NB (inches) | Wall thickness in inches |  |  |
| --- | --- | --- | --- |
|  | 40 Sch | 80 Sch | 120 Sch |
| 5 | 0.258 | 0.375 | 0.500 |
| 6 | 0.280 | 0.432 | 0.562 |
| 8 | 0.322 | 0.500 | 0.719 |