## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Instrumentation-II(S-4216)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the following terms in an instrumentation system with example: i) Sensitivity ii) Resolution. State their units, if any. |  |
| b) | Explain the following terms in an instrumentation system with example: i) Threshold ii) Dead band. |  |
| c) | Explain the function of filter. Sketch frequency response of four kinds of filters. |  |
| d) | Explain the basic concept of v to f converter and state its typical specifications. |  |
| e) | Explain the classification of data presentation elements, with example. |  |
| f) | Explain the working principle and construction of fibre optic transducer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the modes of operation of Piezo-electric crystal. How can force be measured using Piezo-electric crystal? |  |
| b) | Distinguish between moving coil and moving magnet type transducers in terms of principle, construction and features. |  |
| c) | Write a detailed note on ‘absolute encoder’. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept and typical specifications of: - Pneumatic type mechanical amplifier ii) Integrator. |  |
| b) | What is a bridge circuit? Explain Wheatstone bridge circuit with balance equation. Compare null and deflection type. |  |
| c) | Explain the concept and input–output relationship with typical specifications of: i) Charge amplifier ii) DC amplifier. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write note on : i) Rack and pinion gear ii) Differentiator. |  |
| b) | Write note on : i) Electrical tachometer ii) Tachometer encoder. |  |
| c) | Explain the terms: i) Hysterisis ii) Impedance loading and Impedance matching. |  |
| Q.No.5. | Write detailed note on any two: | 2 x 6 = 12 |
| a) | Cathode Ray Oscilloscope |  |
| b) | X–Y plotter |  |
| c) | Servo Recorders |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain the step input response in a zero order, first order and second order system, with appropriate expressions and figures. |  |
| b) | Write note on any one: i) Shielding techniques - Grounding techniques. |  |

***********