## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-I(4055)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the need of clutch in an automobile? List the components of a friction clutch. |  |
| b) | Where and why do we use multiple plate clutch? |  |
| c) | Give any three functions of transmission. |  |
| d) | Explain the necessity of a differential. |  |
| e) | What is ‘ABS’ braking? |  |
| f) | Give a neat sketch of a mechanical clutch linkage. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the loads coming on the chassis frame. |  |
| b) | List two important functions of frame. With a sketch describe the unitary frame. |  |
| c) | Give the advantages and disadvantages of engines fitted at the front and drive given to the rear wheel. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give a neat sketch of a friction clutch plate employing both the cushioning and the torsional springs and describe it. |  |
| b) | Explain with the help of a neat sketch a semi-centrifugal clutch and its advantages and disadvantages. |  |
| c) | List and describe the common clutch facing material. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Using graphs, explain clearly the necessity of a transmission in a vehicle. |  |
| b) | Sketch the constant mesh transmission. Describe the advantages of a constant mesh gearbox over the sliding mesh type. |  |
| c) | Describe the gear shifting mechanism with gear lever above the transmission case. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of universal joint. |  |
| b) | With a neat sketch describe the full-floating axle. |  |
| c) | Explain with sketch the Hotchkiss drive. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the different braking systems. Explain the Air-brakes working. |  |
| b) | Explain how bleeding of brakes is done. |  |
| c) | With a neat sketch explain the construction & working of Master cylinder. What is Tandem Master cylinder? |  |

***********