## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./EC/MED/EI/COMP/E/E&EE

Subject: Basic Electronics(4132)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw an atomic structure of Silicon, Phosphorous and Aluminium. |  |
| b) | What do you mean by intrinsic semiconductor? Give two examples. |  |
| c) | What do you understand by the term depletion region in PN junction? What is the effect of bias voltage on this region? |  |
| d) | What are the different configurations of transistor? |  |
| e) | State any three transistor biasing techniques. |  |
| f) | Define dynamic drain resistance, amplification factor and transconductance of JFET. |  |
| g) | Differentiate between vacuum tubes and transistors.(At least three points) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the static characteristics of Zener diode. |  |
| b) | Explain the working of half wave rectifier with output waveforms. |  |
| c) | Calculate maximum dc voltage available from half wave rectifier shown in fig.1. Also, find current through the load. What is the PIV rating of diode? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how P-type semiconductor is formed? |  |
| b) | What do you mean by filter circuit? Explain the operation of capacitor filter with neat output waveforms. |  |
| c) | Explain PN junction without biasing. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of NPN transistor with neat diagram. |  |
| b) | Explain potential divider biasing circuit with neat diagram. How it is advantageous than other circuits? |  |
| c) | - What causes collector current to flow when the emitter current is zero? What is this collector current called? - Explain the reason why the base current in a transistor is usually smaller than IE & IC in active region. Sketch typical CE input characteristics curves for NPN transistor & label all variables. How to calculate input dynamic resistance of transistor? |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the selection of operating point of transistor. |  |
| b) | In NPN transistor, dc = 0.995, IE = 10mA, ICo = 0.5 A. Determine IC, IB, dc & ICEO. |  |
| c) | Sketch the basic construction of N-Channel JFET. Draw its symbol. Explain how it conducts. |  |
| d) | Explain thermal runaway of transistor. Also, explain the use of heat sink to avoid the above problem. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the triode parameters: - Plate resistance rp - Mutual conductance gm - Amplification factor  Also show the  = rp x gm. |  |
| b) | Explain the working of vacuum diode. |  |
| c) | Explain Enhancement MOSFET with the characteristics curves. |  |