## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Tool Engineering-II(Press Tools)[5077]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following: |  |
| --- | --- | --- |
| a) | Design and the draw the simple blanking die to blank the work piece shown in figure 1 giving: - At least two views of assembly - Detailed drawings of punch and die block (2 views each) - Press tonnage and stripping force calculations - Bill of material. (Take shear strength of sheet = 410 N/mm2) ### OR | (16) (08) (06) (05) |
| b) | Design and draw a simple piercing die to pierce two holes in the work piece shown in figure 1, giving: - At least two views of assembly - Detailed drawings of punch and die block (Two views each) - Die clearance calculations - Bill of material (Take shear strength of sheet = 410 N/mm2) | (16) (08) (06) (05) |
| Q.No.2. | Answer any three of the following Questions: | 3 x 8 = 24 |
| a) | Give two different strip layouts for blanking the work piece shown in figure 1. Assume the length of strip to be 1250 mm. Also calculate percentage of scrap generated in each case. |  |
| b) | State the advantages of progressive die. How does a progressive die differ from a compound die? Explain giving sketch. |  |
| c) | Describe the following components of a cutting die: (any two) (i) Punch (ii) Die block (iii) Pilots (iv) Stripper. |  |
| d) | Explain drawing operation giving neat sketch. Also state the effects of too large and too sharp draw radius on die. |  |
| e) | Describe various methods of mounting pilots on punch, giving neat sketches. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 8 = 16 |
| a) | What is bend allowance? Calculate blank length for the work piece shown in figure 2. |  |
| b) | What is spring back? Explain different ways of preventing or handling spring back. |  |
| c) | Describe following forming processes: (i) Coining (ii) Embossing. |  |
| d) | Write short notes on: (i) Stretch forming (ii) Hydroforming |  |

………2/-