## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: Power Electronics & Drive Control(5102)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the characteristics of “DIAC”. State any two applications of “DIAC”. |  |
| b) | Why are power diodes connected in series and parallel? |  |
| c) | Explain the function of a free wheeling diode with a neat diagram. |  |
| d) | Draw the diagram of a Snubber circuit for an SCR. How does it provide dv/dt protection? |  |
| e) | Draw the circuit diagram and relevant waveforms for a single phase half wave converter with RL load. |  |
| f) | Differentiate between ON-OFF controllers and phase angle controllers. |  |
| g) | What is an inverter? List out atleast 2 applications of inverter. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of a thyristor firing circuit using PUT with circuit diagrams and waveforms. |  |
| b) | Sketch the VI characteristics of a thyristor and explain it in detail. |  |
| c) | Describe the series and parallel operation of thyristor with necessary sketches. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram and waveforms explain the operation of a single phase dual converter. |  |
| b) | With a neat diagram and waveforms, explain the principle of ON-OFF control in AC voltage controllers. |  |
| c) | Write a short note on AC voltage controllers with PWM control. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how it is possible to recover the trapped energy with the help of a diode. Draw the relevant waveforms and circuit diagram. |  |
| b) | With the help of a diagram, explain the principle of operation of inverter. Also list the classifications of inverters. |  |
| c) | What do you understand by a chopper? Give its classification and explain step down chopper circuit with necessary diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram and waveforms, explain the working of a three phase dual converter. |  |
| b) | Write a short note on power control method used in chopper controlled drives. |  |
| c) | Classify DC drives. Also explain in brief any one of such classified DC drives of your choice with the help of neat circuit diagram and waveforms. | …..2/- |

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What is slip power? With the help of neat circuit diagram explain how is it possible to recover slip power by using appropriate method using thyristorised devices. |  |
| b) | Write a short note on “Frequency control method”. |  |
| c) | Write a short note on “Voltage and Frequency control”. |  |

***********