## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-III(5050)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain what is dual fuel engine. Why has it gained importance in recent years? |  |
| b) | List at least three advantages of leaner overall Air-Fuel mixture in I.C. Engine. |  |
| c) | What is “Smog”? Also list I.C. Engine pollutants. |  |
| d) | List actuators used in the operation of the MPFI system and explain any one. |  |
| e) | Give three points of difference between Fuel Injection in Diesel Engine and Petrol Engine. |  |
| f) | List and explain disadvantages of MPFI system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a few lines on i) Regenerator and ii) Gasifier used in Chrysler free shaft gas turbine. |  |
| b) | With a neat sketch explain the configuration of automotive gas turbine power plant. |  |
| c) | Compare I.C. Engine with the Gas Turbine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the suitability of the following engines as multi-fuel engines: - Two-stroke opposed piston engine - Variable compression ratio engine. |  |
| b) | List the advantages and disadvantages of sterling engine. |  |
| c) | Explain the construction and working of Wankel Rotary engine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any three methods to control I.C. engine emissions by controlling the Air- Fuel mixture. |  |
| b) | Describe the EGR method with a neat sketch. |  |
| c) | Explain what is ‘diesel smoke’, its caused and control. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Using a neat sketch explain the construction and working of an MPFI injection valve. |  |
| b) | With a neat sketch explain the BOSCH L-type petrol Injection system. |  |
| c) | Explain with a block diagram the working of two types of sensors used in the MPFI system. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare between petrol and diesel engine exhaust emissions. |  |
| b) | Explain the Air Injection method of exhaust emission control. |  |
| c) | Distinguish between TBI and MPFI system. |  |

***********