## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(4108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State advantages of AC transmission over DC transmission. |  |
| b) | Explain in brief why the SP switch is to be provided in live wire and not in neutral wire. |  |
| c) | What do you mean by demand factor and diversify factor? |  |
| d) | What do you mean by stay or guys? Why are they required? |  |
| e) | State the factors affecting earth resistance. |  |
| f) | List the types and size of conductors used in rural electrification. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive and compare the volume of conductor material required in DC 2 wire system with AC single phase system. |  |
| b) | Briefly explain the factors to be taken into consideration for selection of size of conductors for feeders and distributors. |  |
| c) | A single phase distributor has total resistance of 0.4 and 0.6. At the mid point a current of 100 A at 6.0 p.f. lagging is tapped, at the far end a current of 80A at 0.8 p.f. is tapped. If voltage at supply end is 230V, find voltage at the load point. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the different components of a distribution line and state function of each component in brief. |  |
| b) | Estimate the main material required for 750m, 415V 3 phase, 4 wire line in vertical configuration. The line starts from substation to feed a load of 50KW. The size of conductor used is 6/1 x 2.59 mm. Assume suitable span between poles. |  |
| c) | Explain SWER system for rural electrification. State its advantages and disadvantages. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain various types of faults in the system and their effect on the distribution system. |  |
| b) | State the advantages and disadvantages of PVC casing and capping type wiring. |  |
| c) | A classroom of 5m x 5m x 4m requires 4 tube lights and two fans. Tubelight rating is 40W and that of fan is 60W. For PVC casing capping wiring system estimate total quantity of material required. Also draw a simple layout showing location of switchboard, fans and tubes. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Write a short note on ELCB. |  |
| b) | A small workshop of 15m x 10m has to be equipped with following motors: - 20 HP 400V 3 IM - 10 HP 400V 3 IM Draw a wiring diagram and determine the main switch rating and motor switch rating. |  |
| c) | With a neat diagram explain pipe earthing. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the methods of reducing earth resistance of the system. |  |
| b) | With the help of neat diagram explain ring and radial distribution system. |  |
| c) | Discuss the IE rules for domestic wiring system. |  |

***********