## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Design of Structures-I(Steel)[4024]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

- Use of Steel Tables and IS800-1984 code is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With a neat sketch define the following terms in fillet welds: - Effective throat thickness - Size of weld - Overlap. |  |
| b) | State the applications and merits and demerits of steel structures. |  |
| c) | State the necessity of lacing system in compression members. Also draw a neat sketch of lacing system. |  |
| d) | State the different types of loads considered in the design of steel structures. |  |
| e) | State the components of plate girder with a neat sketch. |  |
| f) | Define slenderness ratio and its significance in compression members. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a fillet welded joint for connecting two plates of 125 x 8 mm and 125 x 10 mm size. Take shear stress as 108 MPa and tensile stress as 150 MPa. |  |
| b) | With a neat sketch explain the procedure involved in the design of eccentric welded joint for connecting bracket plate to column subjected to moment in the plane of welds. |  |
| c) | Design fillet welds for connecting a tee section ISNT 100 @ 196.2 N/m connected to gusset plate of 10mm thick. Take shear stress in weld as 110 MPa and strength of member 150 KN. |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | Design a tie member using single angle (unequal) section to carry a pull of 160 KN. Also design fillet welds. Take shear stress in weld as 108 MPa and tensile stress 150 MPa. | (8) |
|  | ### OR |  |
| a) | A tension member consists of 2 ISA 90x60x8mm connected to gusset plate by its longer legs. Determine tensile capacity of member in the following cases: - Longer legs back to back on same side of gusset plate without tack welds. - Longer legs back on different side of gusset plate with tack welds. | (8) |
| b) | Bracing member of a truss consists of ISA 75x50x8 connected to gusset plate by welds. Determine tensile strength of member. | (4) |

………2/-

-2-

| Q.No.4. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | A discontinuous strut consists of following sections. Determine safe compressive strength of members. - Single angle section ISA 80x80x8 mm connected by fillet welds to gusset plate with length of member as 1.8m. - 2 ISA 125x95x10mm placed on either side of gusset plate of 10 mm thick with length of member as 2.5m connected by fillet welds. | (8) |
|  | ### OR |  |
| a) | A steel column of I section is to be designed to carry a load of 900 KN with length of member as 6.5m. Use the following support conditions. Major axis both fixed and minor axis one end fixed other as hinged. | (8) |
| b) | Draw neat sketches of various types of sections used as struts and built up column. | (4) |
| Q.No.5. | Answer the following Questions: |  |
| a) | The floor beams provided at 4.5m apart simply supported over effective span of 6 m. The top chord of beam are encased in R.C.C. slab of 150mm thick. The floor slab supports an imposed load of 4 KN/m2. Design the beam and carry out check for shear and deflection. | (8) |
|  | ### OR |  |
| a) | A simply supported beam with compression flange laterally restrained for an effective span of 6.5m is ISMB 300 @ 433.6 N/m. Find the safe Udl carried by beam and carryout check for shear and deflection. | (8) |
| b) | Briefly explain design procedure considered in web buckling check of beams. | (4) |
| Q.No.6. | Answer the following Questions: |  |
| a) | Check the suitability of ISA 125x75x6mm angle to be used as purlin for the following data. Spacing of trusses : 4m c/c; Spacing of purlin : 1.6m; Weight of roofing material : 162 N/m2; Slope of roof : 26; Wind load normal to roof as 1200 N/m2. | (8) |
|  | ### OR |  |
| a) | Select a suitable truss for effective span of 12 m to serve as a roof for a Gymkhana building. The roof coverings used is AC sheets of 170 N/m2. Determine the dead loads and live loads at panel points of roof truss. Rise of truss is ¼ of span. Draw dead load and live load diagram of truss selected. | (8) |
| b) | Briefly explain design procedure of anchor bolts at the support of roof truss. | (4) |

***********