## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Shopfloor Practice-I(4333)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the difference between live centre and dead centre. Where are they located? |  |
| b) | State the general safety precautions and maintenance of a lathe. |  |
| c) | Why balancing is done? Suggest a suitable method of balancing a grinding wheel. |  |
| d) | How do you classify the different types of grinding machines? |  |
| e) | How do you specify the drilling machine? |  |
| f) | How the cutting speeds and feeds of a drilling machine is calculated? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the brief the general safety precautions and maintenance required while doing the work on boring machine. |  |
| b) | With a neat diagram explain the working of a 3 jaw self centering chuck. |  |
| c) | Discuss the brief general safety precautions and maintenance of a lathe machine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of a neat diagram different parts of drilling machine and state their functions. |  |
| b) | Explain with the help of neat diagram tool holding devices and work holding devices used in boring machine. |  |
| c) | Describes different types of tools used on boring machine and state the main features of a boring machine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief surface preparation required for spray painting. |  |
| b) | Explain in brief meaning of three dimensional bending for ship construction. |  |
| c) | Explain the working principle of plate rolling machine and state its use in shipbuilding. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief, how does a sensitive drill differ from an upright drilling machine. |  |
| b) | Describe the use of a face plate and show a set up having a face plate, dog carrier and both the centres on a lathe. |  |
| c) | Discuss in brief the factors involved in heating and bending applications. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Equipment required for painting |  |
| b) | Safety precautions of drilling machine |  |
| c) | Boring machine mechanism |  |
| d) | Drilling machine maintenance |  |
| e) | Tool holding devices in drilling machine |  |
| f) | Specification of Centre lathe |  |

***********