## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg.

Subject: Generation & Transmission(4114)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the importance of daily load curve? |  |
| b) | Briefly discuss the function of economiser in a steam power plant. |  |
| c) | What do you understand by “regulation in transmission line”? |  |
| d) | What is port type insulation? Give an application where it is used. |  |
| e) | List the non conventional energy sources used for generating electricity. |  |
| f) | List any 3 advantages of interconnected grid system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How is solar power generated? |  |
| b) | The load on a 125 MW power plant in a typical day is as under: Time(hrs); 12-5am; 5-9 am; 9-6pm; 6-10pm; 10-12pm. Load(MW); 20; 40; 80; 100; 20. i) Plot load duration curve & energy curve. ii) Find plant capacity factor, utilization factor & load factor. |  |
| c) | Explain methods of starting diesel engine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the different causes of failure of insulators in a transmission line & explain each in brief. |  |
| b) | Briefly explain methods of improving string efficiency. |  |
| c) | A single phase line has two parallel inductors 3 metres apart. The radius of each conductor is 10 cm, calculate: - The loop inductance of the line per Km - The inductance & capacitance of each conductor per Km. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat block diagram explain nuclear power plant. |  |
| b) | What is corona? State & explain effects of corona. |  |
| c) | Each line of a 3 phase system is suspended by a string of 3 similar insulators. If the voltage across the line unit in 17.5 KV, calculate the line to neutral voltage. Assume that the shunt capacitance between each insulation and earth is 1/8th of the capacitance of the insulator itself. Also find the string efficiency. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A single phase overhead transmission line delivers 1100KW at 33KV at 0.8 p.f. lagging. The total resistance and inductive reactance of the line are 10 and 15 respectively. Determine i) sending end voltage ii) sending end p.f. |  |
| b) | A 100Km long 3 phase 50 Hz transmission line has the following line constants: Resistance/phase/Km = 0.1 Reactance/phase/Km = 0.5 Susceptance/phase/Km = 10x10-6s If the line supplies load of 20 MW at 0.9 p.f. lagging at 66KV at the receiving end, calculate by nominal  method - Sending end voltage - Sending end current. |  |
| c) | Explain how the characteristics of medium transmission line can be determined using nominal T method. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on effect of ice deposition & wind on the sag. |  |
| b) | Describe the vibration of power conductors & explain the methods used to damp out these vibrations. |  |
| c) | Write a short note on stringing of conductors. |  |

***********