## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Electronic Circuits(4135)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw h-parameter equivalent circuit of CE configuration and write equations for input voltage and output current. |  |
| b) | What is the difference between voltage amplifier and power amplifier? Give 3 points. |  |
| c) | What are the advantages of negative feedback in amplifiers? Give 6 points. |  |
| d) | Give 3 points of difference between common source, common drain and common gate configuration of FET. |  |
| e) | What do you mean by transconductance and amplification factor in case of FET? |  |
| f) | Compare CE, CB, CC configuration on the basis of Ai, Av, Ri, and Ro. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate Ai, Av, Ri for CB amplifier with hib= 20, hrb = 3x10-4, hfb = -0.98, hob=0.5x10-6 & RL=1.5K. |  |
| b) | What is the effect of input coupling capacitor and output coupling capacitor on low frequency response of transistor amplifier? Explain using circuit diagram and cut-off frequency. |  |
| c) | What care should be taken while coupling two stages of amplifier? Explain RC coupling with the help of circuit diagram. What is the application of RC coupling amplifier? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw circuit diagram of class B push pull amplifier and explain. What is cross over distortion in class B? |  |
| b) | Draw circuit of complementary symmetry push pull amplifier and explain its working. |  |
| c) | Explain Direct coupling with the help of circuit diagram and its application. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw series current feedback circuit and explain how feedback takes place. |  |
| b) | Draw emitter follower circuit and explain its application. |  |
| c) | Draw RC phase shift Oscillator circuit and explain with the help of waveforms. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw common source FET amplifier circuit and explain. What are its advantages over BJT? |  |
| b) | Draw As table Multivibrator circuit and explain its operation with the help of waveforms. |  |
| c) | Draw Schmitt trigger circuit and explain its working. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | SCR construction and characteristics |  |
| b) | Distortion in amplifiers (any two) |  |
| c) | Hybrid  model of transistor |  |
| d) | UJT construction and characteristics |  |

***********