## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./EC

Subject: Communication Systems-I(4150)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the need for wireless communication. |  |
| b) | List the advantages of using SSB technique. |  |
| c) | Discuss narrowband and wideband FM. |  |
| d) | Differentiate between amplitude and frequency modulation. |  |
| e) | Define Selectivity and Sensitivity of a radio receiver. |  |
| f) | What is the function of an amplitude limiter in an FM receiver? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different types of internal noises in a communication system. |  |
| b) | - Define modulation and explain the need for modulation. - A 400W carrier is modulated to a depth of 75%. Calculate the total power in the modulated wave. |  |
| c) | A sinusoidal carrier voltage of frequency 1200 KHz is amplitude modulated by a sinusoidal voltage of frequency 20 KHz resulting in maximum and minimum carrier amplitudes of 110V and 90V respectively. Calculate (i) Frequency of lower and upper sideband (ii) the unmodulated carrier amplitude (iii) Modulation index (iv) amplitude of each sideband. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a block diagram, explain the working of a Stereophonic FM transmitter. |  |
| b) | Explain with a neat circuit diagram, the suppression of carrier using FET balanced modulator. |  |
| c) | Explain with a neat block diagram and expressions, the phase shift method of sideband suppression. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a block diagram explain Armstrong method of FM generation. |  |
| b) | With the help of a neat block diagram, explain the operation of stabilized reactance modulator. |  |
| c) | Derive the mathematical expression for an FM wave. Also define Carson’s rule. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a block diagram, explain the operations of low-level AM transmitter. |  |
| b) | Explain the working of an AM diode envelope detector. |  |
| c) | With the help of a circuit diagram, explain the working of a balanced slope detector. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of a block diagram, explain the working of a Superheterodyne receiver. |  |
| b) | With the help of a block diagram, explain the working of a Stereophonic FM receiver. |  |
| c) | Write a note on the following: - Super heterodyne tracking - Factors to be considered when selecting the IF |  |

***********