## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Machine Technology(4217)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between “Machine” & “Mechanism”. |  |
| b) | Define Lever. State the Law of Lever. |  |
| c) | State the functions of (i) Air Preheater (ii) Fusible Plug. |  |
| d) | Sketch and explain the “Spherical Pair”. |  |
| e) | Compare Jet condenser with surface condenser. |  |
| f) | Why is a foot valve needed in a centrifugal pump? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch and explain the working of the Third inversion of the Slider Crank mechanism. Also state the industrial application of this mechanism. |  |
| b) | What is the difference between a simple and compound gear train? Explain with a neat sketch. |  |
| c) | A pulley of 60 cm diameter drivers another pulley of 10 cm diameter at a distance of 4m by a crossed belt drive. If the direction of motion of the driver pulley is to be reversed without changing that of driver, find change in length of the belt. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State advantages and disadvantages of fire tube boilers as compared to water tube boilers. Give examples of their types. |  |
| b) | Sketch and explain the compounding of a steam turbine. State its advantages. |  |
| c) | List out the various boiler mountings and explain any four of them in detail with respect to construction, working and function. |  |
| Q.No.4. | Write brief notes on any three: | 3 x 4 = 12 |
| a) | Simple Impulse Turbine |  |
| b) | Protected type flange coupling |  |
| c) | Vane type air compressor |  |
| d) | Classification of Gears |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are boiler accessories? Explain ‘Superheater’ with a sketch. |  |
| b) | With a neat sketch, explain the working of a centrifugal compressor. |  |
| c) | Describe with a neat sketch evaporative type surface condenser. |  |
| Q.No.6. | Write brief notes on any three: | 3 x 4 = 12 |
| a) | Water ring vacuum pump |  |
| b) | Oldham’s Coupling |  |
| c) | Condenser as a Heat Exchanger |  |
| d) | Troubleshooting in a centrifugal pump |  |
| e) | Type of Belt drives |  |

***********