## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Power Plant Engineering(5037)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define load factor, utility factor and diversity factor. |  |
| b) | Differentiate between high head and low head hydroelectric power plant. |  |
| c) | What is regeneration? How it improves the thermal efficiency of a simple open cycle gas turbine plant? |  |
| d) | Give the advantages and disadvantages of stoker firing. |  |
| e) | Why the feed water treatment is necessary before supplying to the boiler? |  |
| f) | Differentiate between mechanical draft cooling tower and natural draft cooling tower. |  |
| g) | What do you understand by nuclear fission? Explain. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Power station supplies the following loads to the consumer. Time in hrs.; 0-6; 6-10; 10-12; 12-16; 16-20; 20-22; 22-24. Load in MW; 30; 55; 60; 35; 80; 70; 40. - Draw the load curve. - Find the load factor of the plant. - What is the load factor of a standby equipment of 20MW capacity if it takes up all the loads above 60MW? |  |
| b) | What are the various factors for selecting a site for hydro power plant? |  |
| c) | Explain the following systems of an Internal combustion Engine Power Plant: i) Air intake system ii) Exhaust system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of block diagram explain the main components of an open cycle gas turbine Power Plant. |  |
| b) | With the help of neat sketch explain the working of spreader stoker. |  |
| c) | With the help of neat diagram explain the mechanical dust collector. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What factors are considered in selecting a site for a high capacity Thermal Power Plant? |  |
| b) | With the help of neat sketch explain two systems of pulverised fuel firing. |  |
| c) | Explain the ion exchange process used in water treatment plant. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat sketch explain the working principle of natural draught cooling tower. |  |
| b) | With the help of neat sketch explain the construction and working of a Pressurized water reactor. |  |
| c) | - What are the advantages of liquid fuel over solid fuel? - What is the necessity of cooling the surface condenser water when a closed system is used? |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Selection of the number and size of unit |  |
| b) | Clarification of hydro-electric Power plant |  |
| c) | Lub oil system in I.C. engine Power Plant |  |
| d) | De-concentration or blow down |  |
| e) | Nuclear fuels |  |
| f) | Necessity of ash handling system |  |

***********