## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Welding Technology-II(4240)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define GTAW process. List the base metals which can be welded by this process. |  |
| b) | What is the need of CO2 gas preheater? |  |
| c) | Explain the methods of starting an arc in SAW. |  |
| d) | List the applications of Flux cored arc welding. |  |
| e) | State the functions of mold shoes in Electroslag welding. |  |
| f) | Explain how circular patch test is carried out. |  |
| g) | List and explain the factors influencing procedure of maintenance welding. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare Argon and Helium as shielding gases. |  |
| b) | Explain pedestal boom manipulator. |  |
| c) | With a neat sketch explain various types of welding torches. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of operation of submerged arc welding with sketch. |  |
| b) | With a neat sketch explain how flux cored electrodes are manufactured. |  |
| c) | State the difference between Electro slag welding and Electro gas welding. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is Thermit welding? Explain various thermit welding mixtures. |  |
| b) | List the advantages and disadvantages of Electron beam welding. |  |
| c) | Explain with sketch seam welding as a Resistance welding process. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the distortion control techniques in cutting. |  |
| b) | With neat sketches explain various stages of Flash butt welding. |  |
| c) | State the principles of good jigs and fixtures. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Heat shrinkage in spot welding |  |
| b) | Type of residual stresses |  |
| c) | Carbon equivalent and its importance |  |
| d) | Quick acting clamps |  |
| e) | Repair of hair-line crack in casting |  |

***********