## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Drawing(4234)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Structural Engineer’s handbook SP:6(1) & IS800.

5) All dimensions in the figures are in “mm” unless specified.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the difference between Fabrication clearance and site clearance. |  |
| b) | Give the values of ‘Ixx’, ‘tw’ and ‘a’ for the steel rolled sections given below. (any two) i) ISMB 400 @ 61.6 Kg/m ii) ISMC 400 @ 49.4 Kg/m iii) ISMB 250 @ 39.2 Kg/m. |  |
| c) | Give the symbols of the following: (any two) - Site HSFG Bolts ii) Open hole Counter-sunk for side iii) Shop rivet counter-sunk near side iv) Site rivet Counter–sunk far side. |  |
| d) | Give significance of the following in weld representation: - No indication of Length dimensions of weld - No indication of size of weld. |  |
| e) | Calculate the weight of the Mild steel plate piece of size 1450 mm x 850 mm x 8.0 mm thickness. |  |
| f) | What is piping flexibility? Why is it required? How it is incorporated in a pipeline. |  |
| g) | Sketch a typical Ridge joint. |  |
| Q.No.2. | Draw the Gusset Plate at joint A & B and determine the exact length of the member A-B. Refer Fig.1 NOTE for Fig.1. - All Members used are ISA5050 x 6.0 mm - Gusset plate thickness used is 8.0 mm - 12 mm  snap headed rivets are used. | (12) |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the Isometric view of the portion of the pipeline whose plan is shown in the Fig 2. | (08) |
| b) | Explain the different pipe joints and explain their respective areas of applications. | (04) |
|  | ### OR |  |
| b) | Explain any two pipe fittings with neat sketches and mentions their respective functions. | (04) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch a double strap double riveted (chain) butt joint, joining two plates of thickness 8.0 mm and 10.0 mm together. Indicate dimensions clearly. |  |
| b) | Sketch a column splicing, joining bottom column ISMB450 @ 72.4 Kg/m and top column ISMB 125 @ 13.0 Kg/m. Indicate dimensions clearly. |  |
| c) | Give the weld representations of the following: - Fillet weld of throat thickness 6mm, to be done on the other side of the arrow, all along the length of the joint and to be performed in the shop. - A double Bevel butt joint 8.0 mm deep all along the length and to be performed at site by MMAW. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Draw the Orthographic views of the steel structure whose pictorial view is shown in Fig 3. NOTE for Fig.3. - All members used are ISA6060 x 6.0 mm. - All joints are welded Butt joints. - No Gusset plates are used at the joints. | (08) |
| b) | Make a bill of materials for the steel structure shown in fig. 3. Q.5.a. | (04) |
| Q.No.6. | Write a note on any three: | 3 x 4 = 12 |
| a) | Difference between Fabrication Drawing and Design Drawing |  |
| b) | Balancing the weld length in welding Gusset plate |  |
| c) | Specifications of standard steel rolled sections |  |
| d) | Importance of piping supports |  |
| e) | Section modulus of a standard steel rolled section |  |

………3/-