## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Machine Design Practice(4045)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the various steps involved in machine design? |  |
| b) | Define “Stress concentration”. Discuss how the stress concentration in a component can be reduced. |  |
| c) | Why are the arms of pulleys tapered from the hub to the rim? Also give the materials used for pulleys. |  |
| d) | A line shaft rotating at 200rpm is to transmit 20KW. The shaft may be assumed to be of mild steel with an allowable stress of 42 MPa. Determine the diameter of the shaft, neglecting the bending moment on the shaft. |  |
| e) | What is the purpose of using a key in a coupling? Explain the two steps involved in analysing failure of keys. |  |
| f) | Draw the different types of tread profiles used in power screws with their nomenclature. |  |
| g) | Discuss the advantages of rolling contact bearings over sliding contact bearings. |  |
| h) | Explain the following terms in springs: i) Free length ii) Spring index iii) Stiffness iv) Active and Inactive coils. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a socket and spigot joint to resist a tensile load of 60KN. Design stresses in tension, compression and shear are 80 N/mm2, 100 N/mm2 and 50 N/mm2. |  |
| b) | Design a suspension link of a structure as shown in the figure 2(b) which is subjected to a load of 100KN. The width of each side link is 50mm. Given t = 100 N/mm2,  = 75 N/mm2, crushing = 150 N/mm2. |  |
| c) | A right angled bell crank lever having one horizontal arm as 500mm and vertical arm as 150mm is acted upon by a load W= 5KN at the horizontal arm. The lever is made up of forged steel having following permissible stresses in tension and compression, t and c = 84 N/mm2 and  = 70N/mm2 in shear. The bearing pressure for the material of pin is not to exceed 10N/mm2. Design the fulcrum pin and the lever. Refer figure 2(c). | …….2/- |
|  | -2- |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | Shaft is supported by two bearings placed 1m apart. A 600mm diameter pulley is mounted at a distance of 300mm to the right of the left hand bearing and this drives a pulley directly below it with the help of a belt having maximum tension of 2.25KN. Another pulley of 400mm diameter is placed 200mm to the left of right hand bearing and is driven with the help of electric motor and belt which is placed horizontally to the right. The angle of contract for both the pulleys is 180 and  = 0.24. Determine the suitable diameter of the solid shaft, allowing safe permissible stress of 63 MPa in tension and 42 MPa in shear for the material of the shaft. Assume torque on one pulley equal to that of the other pulley. Refer figure 3(a). | (8) |
| b) | A rectangular column is 150mm wide and 120mm thick. It carries a load of 180 KN at an eccentricity of 10mm in a plane bisecting the thickness as shown in the figure 3 (b). Find the maximum and minimum intensities of stress in the section. Refer fig 3(b). | (4) |
|  | ### OR |  |
| c) | A bracket made of steel FeE 200 (Syt = 200 N/mm2) and is subjected to a force of 5 KN acting at an angle of 30 to the vertical as shown in the figure 3(c). The factor of safety is 4. Determine the dimensions of the cross section of the bracket. | (4) |
| Q.No.4. | Answer the following Questions: | (12) |
| a) | A welded bracket is shown in the figure 4(a). Determine the size of the weld if the permissible stress is 80 MPa. |  |
|  | ### OR |  |
| b) | A bracket is supporting the travelling crane as shown in the figure 4 (b). There are 4 identical bolts, two at A and two at B. The bolts are made of steel whose allowable tensile stress is limited to 38 N/mm2. Determine the major diameter of the bolts on the basis of maximum principal stress. Assume (dc = 0.8 d). |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a bushed pin type flexible coupling to connect a pump to a motor shaft. The details are: Power transmitted 32 KN at 960 rpm. The overall torque is 20% more than the mean torque. The material properties are as follows: - The allowable shear stress for the shaft and key material is 40 N/mm2 and 80 N/mm2 respectively. - The allowable shear stress for cast iron is 15 N/mm2. - The allowable bearing pressure for rubber bush = 0.8 N/mm2. - The material of the pin is same as that of shaft and key. |  |

………3/-

-3-

| b) | Design a screw jack for lifting a load of 50KN through a height of 400mm. The screw is made of steel and nut of Bronze. The following allowable stresses may be assumed. For Steel - c = 80MPa,  = 45 MPa. For Bronze - t = 40 MPa, Pb = 15 MPa,  = 25 MPa. Coefficient of friction between screw and nut is 0.12. The dimension of the swivel may be assumed proportionately. The screw should have square threads. Design the screw nut and handle. Take allowable Bending stress for steel handle as 150 MPa. |  |
| --- | --- | --- |
| c) | - What is meant by self locking screw? Derive an expression for efficiency of screw. - What is the type of coupling used in the following: - Shafts whose axis are collinear - Shafts whose axis are parallel - Shafts whose axis are parallel but small distance apart. Sketch the couplings and give their applications. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a semi elliptical spring for the following specification. Total load = 160KN, Number of springs supporting the load = 4, Maximum number of leaves = 10, span of the spring = 1m, Permissible deflection = 100mm. Take young’s modulus, E= 200 KN/mm2 and allowable stress in the spring material as 750 MPa. |  |
| b) | Design a compression helical spring to carry a load of 500N with a deflection of 25mm. The spring Index may be taken as 8. Assume the following values of the spring material. Permissible shear stress = 350MPa Modulus of rigidity = 84 KN/mm2 Wahl’s factor k=4c−14c−4+0.615c , where c = spring index. |  |
| c) | - State advantages, disadvantages and applications of following types of Roller contact bearings: i) Deep groove ball bearing ii) Taper roller bearing. - Explain the meaning of the following: i) Preloading ii) Dynamic load carrying capacity. | (3) (3) |
|  |  | ….4/- |