## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-II(4041)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam tables is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any three general forms of nozzle passages with a neat sketch and their uses. |  |
| b) | What is “diagram factor” and “stage efficiency” of a turbine? |  |
| c) | What are the effects of air leakage in condensers? How can it be minimized? |  |
| d) | What is the difference between 2-stroke & 4- stroke engine? |  |
| e) | Mention the advantages of vapour compression refrigeration system over air refrigeration system. |  |
| f) | Draw a neat sketch of closed cycle gas turbine. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Dry saturated steam at 10 bar is expanded isentropically in a nozzle to 0.1 bar. Using steam tables only, find the dryness fraction of the steam at exit. Also find the velocity of steam leaving the nozzle when - Initial velocity is negligible, and - Initial velocity of the steam is 135m/s. |  |
| b) | The velocity of steam, leaving the nozzles of an impulse turbine, is 1200m/s and the nozzle angle is 20°. The blade velocity is 375m/s and the blade velocity coefficient is 0.75. Assuming no less due to shock at inlet, calculate for a mass flow of 0.5 Kg/s and symmetrical blading: - Blade inlet angle - Driving force on the wheel - Axial thrust on the wheel, and - Power developed by the turbine. |  |
| c) | In a surface condenser, the vacuum maintained is 700mm of Hg. The barometer reads 754mm. If the temperature of condensate is 18°C, determine: - Mass of air per Kg of steam: and - Vacuum efficiency. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working principle of a reaction turbine with suitable sketches. |  |
| b) | What is turbine? Distinguish between impulse and reaction turbine. |  |
| c) | Describe with a neat sketch the working of surface condenser. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the necessity of cooling in I.C. engines? With the help of neat sketch explain the water-cooling system used in I.C. engines. |  |

………2/-

-2-

| b) | What is the necessity of cooling tower in a Power plant? With a neat sketch explain any one type of cooling tower. |  |
| --- | --- | --- |
| c) | Explain with neat sketches the working principle of a 4-stroke cycle Petrol engine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare Petrol engine with Diesel engine. |  |
| b) | With a neat sketch explain Magneto-Ignition system used in I.C. engines. |  |
| c) | The following data were recorded during testing of a four stroke cycle gas engine: Area of indicator diagram=900mm2; Length of indicator diagram=70mm; Spring scale=0.3bar/mm; Diameter of Piston=200mm; Length of stroke=250mm; Speed=300rpm. Determine: - Indicated mean effective pressure and - Indicated Power. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch vapour absorption system of refrigeration. |  |
| b) | Compare open cycle gas turbine and close cycle gas turbine. |  |
| c) | What are the various sources, which add heat into the room? Show following air conditioning processes on Psychometric chart: - Sensible heating - Sensible cooling. |  |

***********