## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Materials & Structures(4232)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain concept of Bi-axial and tri-axial stress systems. |  |
| b) | A rod 3m long is at a temperature of 15C. Find the expansion of the rod if temperature is raised to 80C. If this expansion is prevented, find the stresses in the material. E=2 x106 Kg/cm2 =12x10-6/C. |  |
| c) | Show with neat diagram shear stress distribution for a rectangular section. |  |
| d) | What is point of contraflexure? Give an example. |  |
| e) | State and explain Perpendicular axis theorem. |  |
| f) | Explain the terms Resilience, Proof Resilience and Modulus of Resilience in strain energy. |  |
| g) | What are the assumptions made in theory of simple bending. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A round copper rod ABCD 660mm long, has a diameter of 40 mm over a length of 200mm, a diameter of 30mm over a length of 200mm and diameter of 20mm over remaining length. Determine the stresses in each section and elongation of rod when it is subjected to a pull of 40KN. Take E=100 KN/mm2. |  |
| b) | Two copper rods and a brass rod support a load of 35KN as shown in fig.1. The diameter of the copper and brass rods is 25mm. Find the stresses in rods. Take Ec=1.1 x 105N/mm2 Eb= 1.0 x 105 N/mm2. |  |
| c) | Determine the change in dimensions of a rectangular bar of length 2m, width 200mm and thickness 100mm, when it is subjected to a pull of 20KN in direction of its length. Take E=2x105 N/mm2,  = 0.3 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rod length 1m and diameter 20mm is subjected to a tensile load of 20 KN. The increase in length of rod is 0.30mm and decrease in diameter is 0.0018mm. Calculate Poisson’s ratio and the three modulli. |  |
| b) | A member of length 1.5 m and 30 mm diameter is subjected to a suddenly applied load of 25KN. Find the maximum instantaneous stress induced in the member and elongation of member. E= 2 x105N/mm2 |  |
| c) | An unknown weight falls by 50mm on a collar rigidly attached to the lower end of vertical member of 3m length and 20mm diameter. If the maximum instantaneous extension of bar is 4mm, find the unknown weight. Take E= 2 x 105 N/mm2 |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Find the Moment of Inertia of an I-Section with top flange 8cm x 2 cm, web 12cm x 2cm and bottom flange 16cm x 2cm about an axis passing through the C.G and parallel to bottom flange. |  |
| b) | Find the Moment of Inertia of an I-Section having both flanges and web 20 cm x 6 cm, about x-x and y-y axis. |  |
| c) | A simply supported wooden beam of span 1.5 m having cross- section 100mm wide and 250mm deep carries a concentrated load of W at the centre. If allowable bending stress is 10 N/mm2 and shear stress is 1.5 N/mm2. Find the safe load W. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An overhanging beam of length 8m, rests over two supports 5m apart at a distance of 1m from left end. The beam carries an udl of 20 KN/m over its entire length. Draw shear force and bending moment diagram and locate point of contraflexure. |  |
| b) | A cantilever beam of length 2m carries an uniformly distributed load of 3 KN/m over a length of 1.5m from its fixed end and a point load of 5KN at its free end. Draw shear force and bending moment diagram. |  |
| c) | A simply supported beam of length 4 m carries an udl of 3 KN/m over central 2m length and two point loads 2 KN and 3KN at distances, 0.5m and 3.5 m from left support. Draw shear force and bending moment diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A beam is 3m long of section 200mm wide and depth 300mm. What uniformly distributed load per metre, the simply supported beam can carry safely, if bending stress is not to exceed 100N/mm2? |  |
| b) | A beam 250mm wide and depth 400mm is used to carry a uniformly distributed load of 1000N/m over a span of 5m. Find the maximum stress developed in the beam and state nature of the stress. |  |
| c) | A timber beam of 5m length has to carry an udl 17500 N/m and a concentrated load of 5 KN at mid-span. If the permissible bending stress be 10 N/mm2. Find the section taking depth as twice the breadth. |  |