## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(4331)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Catamaran |  |
| b) | Camber and Shear |  |
| c) | Motions of a ship |  |
| d) | Simpson’s 141 rule |  |
| e) | Centre of bouyancy |  |
| f) | Archimedes principle |  |
| g) | GZ Curve |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | A ship of 5000 tonnes displacement has KG=4.5m. The following cargo is loaded: 2000 tonnes KG 3.7m 1000 tonnes KG 7.5m Find how much deck cargo (KG=9m) may now be loaded to have final KG at 5 m. | (08) |
| b) | Explain briefly about Lines Plan. | (04) |
| Q.No.3. | A ship has water plane areas at intervals of 0.9m from the keel as follows: | (12) |
|  | Draught(m); 0; 0.9; 1.8; 2.7; 3.6; 4.5; 5.4. Area (m2); 0; 760; 1000; 1160; 1260; 1340; 1400. |  |
|  | Determine the displacement of the ship in sea water and KB at the draught of 5.4m. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | Explain briefly with sketches - Bonjean Curves - Hydrostatic Curves. | (08) |
| b) | Sketch the profile and transverse view of a ship to explain - Length between perpendiculars - Moulded breadth. | (04) |
| Q.No.5. | Answer the following Questions: |  |
| a) | A vessel is box shaped and has length 32m, breadth 8m and depth of 5m. The KG =3.7m when the vessel is floating at a draft of 4m. Prove that KM=T2+B212T Also, prove that vessel is unstable. | (08) |
| b) | A ship of 2000 tonne displacement has a rectangular bottom tank 4m wide and 9m long, half full of sea water. Calculate the virtual reduction in GM due to free surface. | (04) |

………2/-

-2-

| Q.No.6. | A ship’s water plane is 90m long. The half ordinates at this waterplane are as follows: Stn; AP; 0.5; 1; 2; 3; 4; 5; 5.5; FP. ½ Ord (m); 0; 2.0; 4.88; 6.71; 7.31; 7.01; 6.4; 2.0; 0.9. Determine the metacentric height GMT given the following: Volume of displacement =5024m3, KG=4.6m, KB=3.0m. | (12) |
| --- | --- | --- |
| Q.No.7. | Answer the following: |  |
| a) | Explain body plan with sketch. | (04) |
| b) | A ship of 5000 tonne displacement, 96m long floats at draughts of 5.60m even keel. The TPC is 11.5, GML=105m and centre of floatation 2.4m aft of midships. Calculate i) MCT 1cm ii) New drafts when 88 tonne is added 31m forward of midships. | (08) |

***********