## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./EC/EI

Subject: Electronic Measurements(4137)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the terms ‘accuracy’ and ‘precision’ with example. |  |
| b) | State three characteristics of a good display. State three advantages of digital displays. |  |
| c) | What is the function of delay line in a CRO? What does it comprise of? |  |
| d) | Sketch i) Kelvin bridge ii) Inductance comparison bridge. |  |
| e) | Sketch block diagram of a signal generator. State function of each block. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the block diagram of a measurement system. Explain the following i) any two environmental errors ii) dynamic characteristics (any two). |  |
| b) | Explain the principle and construction of D’Arsonal movement. State its limitations. |  |
| c) | Draw the circuit of an electronic voltmeter and explain its operation. What are its advantages over ordinary analog voltmeter? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How is PMMC modified to carry out resistance measurements? Explain with circuit diagram. Why is resistance scale of analog miltimeters non-linear and inverted? |  |
| b) | Explain the operation of a Wheatstone bridge with diagram and expressions. What are its limitations? |  |
| c) | In Maxwell bridge obtain the expression for unknown impedance measured. State advantaged and disadvantages of such bridge. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How are displays classified? Explain with examples. |  |
| b) | Explain the principle, construction and characteristics of LCD. How does it compare with LED? |  |
| c) | Draw block diagram of CRO. Briefly state function of each block. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the requirements of probes in CRO? Explain the need for probe compensation. State advantages of active probes. |  |
| b) | Sketch Lissajous pattern obtained when two sinusoid signals of same amplitude, with frequency of one being twice that of the other, are fed to the two sets of deflection plates of a CRO. What information can be obtained from Lissajous patterns? |  |
| c) | Write note on: i) dual trace oscilloscope ii) dual beam oscilloscope | ………2/- |

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the operation of a frequency synthesized signal generator with block diagram. |  |
| b) | Describe the operation of AF generator with block diagram. |  |
| c) | What is a sweep frequency generator? Explain its operation. |  |

***********