## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the essential factors in preparation of detailed estimate? |  |
| b) | Differentiate between Revised Estimate and Supplementary estimate. |  |
| c) | Explain cost analysis for water supply, sanitation and electrification in an estimate. |  |
| d) | Explain the terms lumpsum and provisional sums. |  |
| e) | Enumerate importance of valuation in construction industry. |  |
| f) | What is arbitration? Explain importance of Indian Arbitration Act. |  |
| g) | Explain importance of estimation in construction industry. |  |
| h) | Under what circumstances a tender can be rejected? |  |
| Q.No.2. | Answer any one of the following Questions: | (15) |
| a) | Fig No.1 show plan and wall section of a single storied building. Calculate quantities of any five items for the building. (Assume missing data) |  |
|  | - Earthwork in Excavation - Masonry in superstructure - Internal plastering 12mm thick (CM = 1:4) - Flooring using marbles - D.P.C. - Wood work in door/window shutters. |  |
| b) | Using suitable method estimate quantity of earthwork for a road between two stations A to B with following data. One chain = 30m, width of road 10m, side slope in cutting 1.5:1, side slope in Banking 2:1. Draw longitudinal and cross sectional view. |  |
| Chainage; 0; 1; 2; 3; 4; 5; 6. R.L of ground; 123.70; 124.90; 124.60; 122.90; 121.60; 121.00; 120.40. R.L of formation; 123.20; 123.60; 124.00; 123.60; 123.20; 122.80; 122.40. Assume any suitable data, if required. |
| Q.No.3. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Carryout rate Analysis for Manglore tile Roofing over 10m2 area with wooden battens 45mm x 25mm @ 35cm c/c. |  |
| b) | Carryout Rate analysis for laterite stone masonry in cement mortar 1:4 for 10m3 quantity of work. |  |
| c) | What is a contract? Explain essentials to form a contract. |  |
| d) | Write a detailed specification for precise terrazzo tile flooring. |  |

………2/-

-2-

| Q.No.4. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Prepare a tender notice for construction of Educational building costing Rs. 60 lakhs to be completed in 18 months and executed under PWD. |  |
| b) | Explain labour contract and material supply contract stating its advantages and disadvantages. |  |
| c) | Enumerate the factors affecting value of an asset. |  |
| d) | Explain different methods of approximate estimate in buildings. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Give list of tender documents and state their use for owner and contractor. |  |
| b) | What is M.A.S. and temporary advance and also state significance of each one. |  |
| c) | Explain the type of approvals necessary for execution of any building. |  |
| d) | Define the terms Book value, assessed value and potential value. |  |