## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Microprocessor Based Instrumentation(5114)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the term ‘instruction queue’ used in 8086 architecture and state its advantages. |  |
| b) | What is a Microcontroller? State some applications. |  |
| c) | When is the over-flow flag set in 8051 microcontroller? |  |
| d) | Draw the block diagram of 8051 microcontroller. |  |
| e) | Mention at least two instructions of 8051 which use stack memory. Explain one instruction in detail. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is memory segmentation? Explain its advantage with reference to the 8086 processor. |  |
| b) | Explain in detail the general purpose registers and their special functionalities with respect to 8086 microprocessor. |  |
| c) | Explain the different addressing modes of 8086 processor and give examples of each. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the oscillator and clock section of 8051 Microcontroller and define the time taken to execute an instruction. |  |
| b) | Explain “Memory Organization” of 8051 Microcontroller. |  |
| c) | Compare the following: - Microcontroller and Microprocessor - 8-bit Microcontroller and 16-bit Microcontroller. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the modes of operations used for serial data transmission in 8051 systems. |  |
| b) | With the help of port pin circuit diagram, explain input-output operations in 8051 systems in brief. |  |
| c) | Explain the importance of INT0and INT1 interrupts of 8051 microcontroller. How 8051 can be interrupted by external device at INT0 pin? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following instructions along with type of addressing mode. - XCHD A,@Ri - CPL 91H - RLC A |  |
| b) | Assume internal RAM locations of a 8051 as having these bytes: 40 =(7D), 41=(EB), 42=(13), 43=(5B) and 44= (25). Write a 8051 program to search these locations and find if any value equals 30. If value 30 is found, then make R4=(30) and if not, make R4=(00) |  |
| c) | Write a 8051 program to add the byte present in internal RAM location 25H to all the bytes present in RAM memory locations from 00H to 20H. | ………2/- |

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the following along with number of bytes required to code. - ANL A, # N - SJMP <LOOP> - DJNZ R1, <LOOP> |  |
| b) | Write assemble language program to transfer data from external memory location starting from 2000H to 2010H to internal RAM location starting at 104(10). |  |
| c) | Write short note on following: - Call and return instructions - Instructions used for look-up table. |  |

***********