## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Automobile Engg.

Subject: Hydraulic Machinery(4053)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by intensity of pressure and pressure head? State the equation relating them. |  |
| b) | What are Manometers? State the different types of manometers used for pressure measurement. |  |
| c) | State Bernoulli’s theorem. |  |
| d) | Explain i) Total pressure acting on immersed surface ii) Centre of pressure. |  |
| e) | Mention the different types of impellers used in Centrifugal pump with their applications. |  |
| f) | Draw a neat labelled diagram of vane pump. |  |
| g) | Find the pressure at a depth of 50 m below the surface of seawater. Take specific gravity of sea water as 1.03. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive the equation for difference of pressure between two pipes carrying certain liquid and connected by inverted differential manometer with light manometric fluid. |  |
| b) | What is an Air Vessel? Give its various functions in a reciprocating pump. |  |
| c) | Explain the working of hydraulic jack with the help of a neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of centrifugal pump. Why priming is done in Centrifugal pump? |  |
| b) | An isosceles triangular plate of base 4 m and altitude 4 m is immersed vertically in water. The base of the plate coincides with the free surface of water. Find i) Total pressure on the plate and ii) Position of centre of pressure. |  |
| c) | Explain the construction and working of a Gear pump with the help of a neat sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An orifice 50 mm in diameter is discharging water under a head of 8 m. If Cd = 0.6 and actual velocity of the jet at vena contracta is 10 m/s. Find the actual discharge and coefficient of velocity (Cv). |  |
| b) | Find the maximum power that can be transmitted through a pipe carrying water at the rate of 0.5 m3/s under a head of 500m. |  |
| c) | An orifice meter is connected to pipe of diameter 200mm. The diameter of the orifice is 100mm and differential manometer reading is 300mm of mercury. Calculate the flow rate of liquid if specific gravity of oil is 0.9 and coefficient of discharge of meter is 0.64. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Describe the construction and working of hydraulic press with a neat sketch. |  |
| b) | What is separation? Why reciprocating pump cannot be run at high speed? |  |
| c) | Find the discharge of water flowing over a rectangular notch of 2m length when the constant head over the notch is 350 mm. Take Cd = 0.62. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Losses in pipe flow |  |
| b) | Multistage Centrifugal pump |  |
| c) | Dead weight pressure gauge |  |
| d) | Pitot tube |  |

***********