## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./EC

Subject: Television Engineering(5135)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain (i) aspect ratio (ii) flicker. |  |
| b) | Explain the terms (i) Primary colours (ii) Complementary colours. |  |
| c) | Explain briefly grey scale tracking. |  |
| d) | Write any three T.V. standards. |  |
| e) | Write abbreviations for the following: (i) NTSC (ii) PAL (iii) SECAM. |  |
| f) | Differentiate between positive and negative modulation. |  |
| g) | Explain the importance of colour burst signal. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch composite video signal for three successive lines and indicate: - Extreme white level - Blanking level - Pedestal height. Briefly explain the meaning of the above three terms. |  |
| b) | Draw cross-sectional view of an image orthicon camera tube and explain how it develops video signal when light from any scene is focused on its face plate. |  |
| c) | Explain with the help of a suitable sketch, how video signal is developed on Vidicon camera tube. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a simplified block diagram of TV transmitter. State the advantage of using FM for sound transmission. |  |
| b) | Explain how flicker is eliminated in TV systems using interlaced scanning. What is Interlace error? |  |
| c) | Draw and explain the functioning of sound channel in TV system, beginning from separation of intercarrier sound signal from video signal. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat diagram, explain the construction and operation of a delta gun colour picture tube. |  |
| b) | Draw a neat block diagram of the PAL coder and explain the same. |  |
| c) | Why is the (G-Y) difference signal not chosen for transmission? Explain how it is obtained in the receiver. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Compare the features of PAL, SECAM & NTSC TV systems. |  |
| b) | Explain the following: - Pincushion adjustments - Automatic degaussing circuit. |  |
| c) | Define compatibility. How is it achieved? |  |
| Q.No.6. | Write brief note on any four: | 4 x 3 = 12 |
| a) | Co-axial cables |  |
| b) | VSB transmission |  |
| c) | Frequency interleaving |  |
| d) | AFC |  |
| e) | Pre equalizing & post equalizing pulses |  |
| f) | Advantages of AGC |  |

***********