## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-II(4008)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Explain Direct method of contouring with a figure. |  |
| b) | Mention parts of Theodolite & write down their functions. |  |
| c) | Explain principles of Plane table survey & Tacheometric survey. |  |
| d) | With a neat sketch explain notation of simple circular curve. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat figure mention the characteristics of contour. |  |
| b) | Explain any methods of prolonging straight line. |  |
| c) | Define fundamental axes of Theodolite & write down the relationship. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain traversing with a Theodolite. |  |
| b) | An incomplete traverse table is obtained as follows. Line; Length (m); Bearing. AB; 100.0; ?. BC; 80.5; 140 30. CD; 60.0; 220 30. DA; ?; 310 15. Calculate the length of DA & bearing of AB. |  |
| c) | The record of a closed traverse is given below, with two distances missing. Line; Length (m); Bearing. AB; 100.5; N30 30 E. BC; ?; S45 0 E. CD; 75.0; S40 30 W. DE; 50.5; S60 0 W. EA; ?; N40 15 W. Calculate the length of BC & EA. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Radiation method of plane tabling. |  |
| b) | Explain Field procedure for determination of Tacheometric constants. |  |
| c) | Explain Tacheometry & write down advantages & disadvantages of it. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the method of setting curve by offsets from long chord. |  |
| b) | Two straight lines AC & CB, to be connected by a 3 curve, intersect at a chainage of 2760m. The WCB’s of AC & CB are 4530 & 7530 respectively. Calculate all necessary data for setting out the curve by the method of offsets from the long chord. |  |
| c) | Explain Reconnaissance Survey. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Interpolation of contour |  |
| b) | Bowditch’s Rule |  |
| c) | Digital Theodolite |  |
| d) | Field location survey |  |

***********