## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-I(4022)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Differentiate between Determinate and Indeterminate structures with mentioning examples of structures. |  |
| b) | Derive the expression for limit of eccentricity for a square section column for a no tensile stress condition. |  |
| c) | With an analytical solution draw the influence line diagram for a simply supported beam for Bending moment at a section due to rolling load. |  |
| d) | A beam is subjected to normal tensile stress of 75 N/mm2 and shear stress of 30 N/mm2. Find principal stress and a minimum shear stress. |  |
| e) | A thin tube 50mm diameter and 5mm thick is subjected to a torque of 800N-m. Find the angle of twist in a length of 2m. Take C= 80 KN/mm2. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A flitched beam consisting of timber beam 150mm wide and 300mm deep is strengthened by a steel plate 20mm thick and 300mm deep on left side of timber beam. Determine Moment of Resistance of beam if stress in timber is 7 N/mm2. Take Es = 20Ew. |  |
| b) | A timber beam 110mm wide and 220mm deep is strengthened by two steel plates 110mm wide and 20mm thick at top and bottom of the timber section. Find the moment of Resistance of beam if allowable stress in timer is 6N/mm2. Take Es = 17Ew. |  |
| c) | Determine Degree of indeterminacy of following structures. (i) Propped cantilever ii) Portal Frame with both supports as fixed iii) Three hinged Arch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A square column of 800mm x 800mm in section carries a vertical load of 600KN on x-axis at an eccentricity of 200mm. Determine the maximum and minimum stresses in the column. |  |
| b) | A bridge column of 1200mm wide, 2400mm deep carries a vertical load of 1200KN at an eccentric of 400mm from YY axis and 600mm from XX axis in 1st quadrant. Determine stresses provided at all corners of column. |  |
| c) | A masonry pillar of 6m height and 2m x 3m in section is subjected to horizontal wind pressure of 1200 N/m2 on 2m x 6m face of pillar. Take density of masonry as 19.2KN/m3. Determine maximum and minimum stress intensities at the base of pillar. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | At a point in a bracket of column there is a longitudinal bending stress of 80N/mm2 tensile and a transverse shear stress of 45 N/mm2 across both the planes. Determine the magnitude and direction of principal stresses and maximum shear stresses. |  |
| b) | At a point in a elastic material normal stresses of 100 N/mm2 tensile on X axis and 80N/mm2 compressive on Y-axis acts. Determine on an oblique plane of 30 with major stress, Tangential stress, Normal stress and Resultant stress. |  |
| c) | In a two dimensional stress system, an elastic material is subjected to direct normal stresses of 80 N/mm2 and 40 N/mm2 both tensile and a shear stress of 30 N/mm2 across both planes. Using mohr’s circle, find magnitude and direction of principal stresses. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam of 10m span carries an UDL of 30 KN/m throughout the span. Determine maximum shear force and Bending moment at centre of girder using Influence line diagram. |  |
| b) | A Live load of 50 KN/m of length 6m is rolling on the girder of span 20m. Calculate the shear force both positive and negative at a section of 8m from left end of girder. Use ILD concept. |  |
| c) | A simply supported girder has span 15m. An UDL of 60 KN/m for a length of 6m is rolling on the girder. Determine the maximum Bending moment at a section of 6m from left end of girder use ILD concept. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A solid steel shaft of 60mm diameter transmits 150 KW power at 80 rpm. Determine shear stress and also angle of twist in a length of 2m shaft. Take Rigidity modulus of shaft material as 80 KN/mm2. Maximum torque exceeds mean torque by 25%. |  |
| b) | A hollow shaft of diameter ratio d/D = 0.6 is required to transmit torque of 3.60 KN-m. If shear stress is limited to 60 N/mm2 determine the diameter of shaft. |  |
| c) | - State the assumptions made in theory of pure torsion. - Derive expression for power transmitted by shaft. | (2) (4) |

***********