## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Welding In Shipconstruction-II(4337)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the two board classifications of a welding fixture. |  |
| b) | Which welding technique is very successful in arc welding of Cast Iron? Discuss. |  |
| c) | List down the advantages and disadvantages of MIG Welding. |  |
| d) | Explain the side bend test of a welded specimen. |  |
| e) | What are the various types of welded joints? Discuss. |  |
| f) | What are the various factors involved in estimation of welding costs? |  |
| g) | List down the steps involved in hard facing a worn out component. Also mention some of the hard facing alloys. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the SAW process with a neat diagram. |  |
| b) | As per the American Bureau of Shipping, explain the Welding Procedure Qualification test. |  |
| c) | Explain the following w.r.t. dissimilar welding process: - Use of prefabricated joining process - Use of a solid intermediate metal. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on welding of Martensitic Stainless Steels. |  |
| b) | Explain the Izod impact test of a welded specimen. Also mention its significance. |  |
| c) | Explain the Dye Penetrant test with its application w.r.t. Shipbuilding Industry. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 120mm diameter solid shaft is joined by 20mm fillet weld to a flat plate. Determine the maximum torque that the welded joint can sustain if the permissible shear stress in the weld material is not to exceed 100N/mm2. |  |
| b) | Two 1 m long MS plates 10mm thick are to be welded by a lap joint with the help of a 6mm electrode. Assuming the following data, make the calculations for the cost of welding. Current used = 250 Amps Voltage = 30 Volts Welding speed = 9 metres/hour Electrodes Used = 0.5 Kg/m of welding Labour charges = Rs. 20/hour Power Charges = Rs. 1.2 per KWhour Cost of Electrode = Rs. 30 per Kg Machine Efficiency = 70% |  |
| c) | What are the problems encountered in performing Underwater welding? Also discuss in brief ‘Wet Welding’ & ‘Dry Welding”. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Discuss the process of cladding of shafts with a neat diagram. |  |
| b) | What are the safety precautions to be undertaken while performing the Gas welding operation? |  |
| c) | Explain in brief the different welding positions. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Welding of Cat Iron |  |
| b) | Preheat treatment & Post heat treatment of Welds |  |
| c) | Magnetic Particle Inspection |  |
| d) | Classification of Direct & Indirect cost in Welding |  |

***********