## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Machine Drawing-II(4035)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Attempt any one of the following Questions: |  |
| --- | --- | --- |
| a) | Fig No.1 shows the details of a ‘SCREW JACK’. Assemble the parts and draw the following: |  |
|  | i) Half sectional Front view (Right half in section) | (10) |
|  | ii) Top view. Use first angle method of projection and insert important dimensions. | (05) |
| b) | Fig. No. 2 shows the assembly of a steam engine connecting rod end. Draw the details of following parts: |  |
|  | i) Strap (Full sectional front view) | (05) |
|  | ii) Brasses (Half sectional front view and top view) | (06) |
|  | iii) Gib and Cotter (Front view). Insert important dimensions. | (04) |
| Q.No.2. | Attempt any four of the following Questions: | 4 x 3 = 12 |
| a) | Calculate the limit size for a hole designated as 25H7, using the following data: |  |
|  | i) Diameter 25mm lies in the range of 18 to 30mm. |  |
|  | ii)Unit tolerance i=0.001D+0.453D microns |  |
|  | iii) D = Mean diameter = D1×D2 |  |
|  | iv) IT7=16i |  |
| b) | Indicate the roughness value Ra in microns for the following roughness grade number. |  |
|  | i) N1 ii) N4 iii) N12. |  |
| c) | Interprete the meaning of Geometrical tolerances of the parts shown in Fig No. 3(a), 3(b) and 3(c). |  |
| d) | Sketch the orthographic sectional front view of the following pipe fittings: i) Elbow ii) Cross. |  |
| e) | Draw neat free hand sketch and the symbol for the following: |  |
|  | i) Cross sectional view of Double ‘J’ butt joint |  |
|  | ii)Cross sectional view of Double bevel butt joint. |  |
| Q.No.3. | Draw neat proportionate free hand sketches of any two of the following: | 2 x 6 = 12 |
| a) | Full sectional front view and side view of Gib and Cotter Joint. |  |
| b) | Full sectional front view and side view of ‘Oldham coupling’. |  |
| c) | Full sectional front view of ‘V-belt pulley’. |  |
| Q.No.4. | Fig No.4 shows the two orthographic views of the base plate of a ‘Drill Jig’. Redraw the views and incorporate following details. | (12) |
|  | i) Tolerance of g6 on thickness of 25mm. |  |
|  | ii) Bottom face of the base plate to be indicated as datum A. |  |
|  | iii) Top face of the base plate is parallel to bottom face and to be within a tolerance of 0.02 mm. |  |
|  | iv) Roughness on the inner surface of hole of diameter 54 mm to be indicated by . |  |
|  | v) Axis of tapped hole of M12 to be co-axial with the axis of hole of 17 mm within  0.08 mm. | ……2/- |

-2-

| Q.No.5. | Attempt the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Fig.No.5 shows the orthographic views of a Bracket to be fabricated by welding. Redraw the views to an appropriate scale and indicate necessary welding instructions in the form of standard welding symbols. |  |
| b) | Fig No.6 shows the Isometric view of a pipeline. Draw the single line orthographic views – Front view and top view. |  |
| Q.No.6. | Study and answer the following from the drawing of ‘CONNECTOR’ shown in Fig.7. | (12) |
|  | i)What is the significance of the length of the connector 165 mm shown within brackets? |  |
|  | ii)What is the size of the knurled portion of the connector? |  |
|  | iii)What is the size of the Key way? |  |
|  | iv)What is the depth of the drilled hole? |  |
|  | v)Explain 50 f 7. |  |
|  | vi)Explain SR 5. |  |
|  | vii)What is the counter bore specification at the internal thread coupling? |  |
|  | viii)What type of tolerance is given on  16? |  |