## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Elementary Ship Design(5331)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five of the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | GRT/NRT |  |
| b) | Dead weight |  |
| c) | Admiralty coefficient |  |
| d) | Crain capacity and bale capacity |  |
| e) | A basis vessel in ship design |  |
| f) | Typical ship owner’s requirements |  |
| Q.No.2. | Estimate the main dimensions, dead weight, displacement, CB of a 30m long fishing trawler, with endurance of 14 days and catch of 20 tonnes. The total power of the vessel is 490 BHP. The fuel oil consumption is 155 gms/BHP/hr and lub oil consumption is 3gms/BHP/hr. It is required to carry 25% extra fuel and lub oil. For the stipulated endurance, fresh water required is 1 tonne/person, provision required is 0.12 tonne/person, crew and effects is 0.15 tonne/person. | (12) |
|  | Total No of crew is 10. Given that dwt/ = 0.22, L/B = 4.2 - 4.5 B/T = 2.15 - 2.68 T/D = 0.7 CB = 0.5 - 0.54 |  |
| Q.No.3. | Estimate the lightship weight of a vessel of service speed of 15 knots of dimension 140m x 20m x 13.2m given the following: | (12) |
|  | - Hull weight of a basis vessel of dimension 135m x 19m x 12m is 5000 tones. - Wood and outfit weight of the basis vessel is 240 tones. - Block co-efficient of basis vessel is 0.78 and that of proposed design is 0.80. - Speed of the basis vessel is 14.5 knots with engine power of 8500 BHP. - Machine weight = Engine BHP/10 + 180 tones. |  |
| Q.No.4.a) | Explain how the hull form of a vessel is obtained from the section area curve in association with the load waterline curve. | (12) |
|  | ### OR |  |
| b) | Explain the various stability requirements of IMO that a vessel should fulfill. Explain the significance of the stability requirements. | (12) |
| Q.No.5. | Draw a typical General Arrangement plan of a cargo vessel. Explain the layout of the various machinery spaces and fittings on board the vessel. | (12) …….2/- |

-2-

| Q.No.6a) | What is freeboard of a vessel? Is it a statutory requirement? Explain broadly how the minimum freeboard is calculated. | (6) |
| --- | --- | --- |
| b) | Explain a typical technical specification and contract signed between the ship owner and shipyard for construction of a vessel. | (6) |
| Q.No.7a) | Explain the stages of design with respect to design spiral. At what stage the general arrangement and lines plan prepared and why? | (6) |
| b) | How do canals, bridges, waterways and ports affect the feasibility study for ship design? | (6) |

***********