## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-II(4023)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain briefly the limitations of Euler’s formula for columns. |  |
| b) | What duo you mean by the distribution factor & carry over factor? |  |
| c) | Determine the central deflection of SS beam, 4m span having a self weight of 0.5KN/m & E=2x105N/mm2. I = 4x104mms4 |  |
| d) | Draw the deflected shape of (i) Cantiliver with point load of free end (ii) Cantiliver with a point load at the mid span. |  |
| e) | Determine the FEM of a fixed beam of span 4m having a central point load of 5KN. |  |
| f) | Using moment area method, find slope of a SS beam with central point load. |  |
| g) | What is distribution factor? How does carryover factor affects the support? |  |
| Q.No.2. | Answer any one of the following Questions: | (12) |
| a) | A horizontal steel girder with uniform c/s is 14m long & is simply supported at its ends. It carries 2 point loads of 12 KN & 8 KN at 3m & 9.5m from the left support. Calculate the maximum deflection & maximum slope. E=200 GPa & I=160x106 mms4. Draw the deflected shape. |  |
| b) | A cantiliver of span 4m is propped at the free end. A point load of 20 KN is acting at its centre. Calculate the reaction of the prop. Determine the maximum slope & deflection. Draw the BMD. |  |
| Q.No.3. | Answer any one of the following Questions: | (12) |
| a) | A fixed beam AB of span 6m, is carrying a udl of 4 KN/m on the left half of the span. Determine the fixed end moments and Draw SFD & BMD. |  |
| b) | A fixed beam of span 7m is carrying two point loads of 10 KN & 20KN at 2m & 4m from left & right support respectively. Determine the Fixed End Moments and Draw SF & BMDS. Take EI=1200 KN-m2. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A hollow cylindrical CI column of 150mm external diameter & 15mm thickness, 3m long is hinged at one end, fixed at the other. Find the load carrying capacity of this column using Euler’s formula. E=8x104N/mm2. |  |
| b) | Solve the above problem (Q.4a) using Rankine’s method. Take  = 1 and fc = 550N/mm2. 1500 |  |

………2/-

-2-

| c) | Determine the limiting length of a pin ended column of section 600mm2 section so that the critical stress due to load (direct stress) would not exceed 250N/mm2. Take E=2x105N/mm2, I=4.5x105 mms4. |  |
| --- | --- | --- |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A continuous beam ABC, fixed at A, consists of span AB=4m & span BC=6m. Span AB caries a udl of 20KN/m while span BC carries a load of 10KN at 2m from C (support). Draw the SFD & BMD after analyzing the beam. Use Moment distribution. |  |
| b) | A continuous beam ABCD of span AB=4m, BC= 4m & CD=4m has udl of 3 KN/m on BC & a central point load of 5KN on span AB & CD. Draw SFD & BMD. |  |
| Q.No.6. | A 3 hinged parabolic arch of span 20m & rise 4m carries a u.d.l of 20KN/m run on the left half of the span. Find the maximum Bending moment for the arch. | (12) |

***********