## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mining Engg.

Subject: Mine Surveying(5396)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Enumerate classification and sources of error. |  |
| b) | Enumerate various methods of correlation. |  |
| c) | List out instruments used in stope survey. |  |
| d) | State application of EDM in mines. |  |
| e) | Draw convention for preparation of mine plan as per second schedule CMR 1957 - Surface contour - Pillar & galleries - Survey lines and stations. |  |
| f) | Give formula of Trapezoidal rule and Simpson’s rule for calculation of area. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Explain shaft plumbing method in correlation. | (5) |
| b) | Explain one wire in each shaft method of correlation. | (5) |
| c) | Enumerate methods of stope surveying. | (2) |
| Q.No.3. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Purposes of stope surveying |  |
| b) | Stope surveying in cut and fill method |  |
| c) | Various methods of plotting a survey |  |
| d) | Preparation and preservation of mine plans |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | State causes of inaccuracy of mine plans. |  |
| b) | Explain role of mine surveying. |  |
| c) | Explain method of chord and angle for underground curve setting. |  |
| d) | State use of Theodolites in open cast mines. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Enumerate various methods of Area calculation. | (4) |
| b) | The following particular offsets were taken at 5 m intervals from a survey line to an irregular mine boundary 3.82, 4.37, 6.82, 5.26, 7.59, 8.90, 9.52, 8.42, 6.43 m. Calculate the area in square metres enclosed between survey line and boundary and first and last offset by - Simpson’s rule - The trapezoidal rule - The average ordinate rule. | (8) |
|  | ### OR |  |
| b) | Calculate the weight of coal heap, which occupies area measuring 27m by 18m and 6m heights. The side of heap slope at 45 and the coal weighs 1 tonne per 1.2 cubic metre. | (8) |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | In a seam of coal the direction of full dip is S45E and rate of dip is 1 in 6. Find the inclination of two slope heading the azimuth of which are 185 and 340. |  |
| b) | In a seam dipping at 1 in 8 one up throw fault is met with. The fault hades at 30 and has throw the seam up by 14.4 metres. Calculate the length of rising drift at l in 4 driven to meet the seam on the other side of fault. Assume that direction and rate of dip of seam remain same on the other side of the fault. |  |
| c) | A level road strike an upward throw fault running on the lines of full dip. The vertical displacement of the fault is known to be 22.5m and hade 1.5 vertical to 1 horizontal. Calculate the length of stone drift rising in 1 in 5 on the line of road to strike the seam at upper side of the fault. Find also position of the starting point. |  |

***********