## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(4104)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | For the circuit shown in fig.1, find the equivalent resistance across terminals A & B. |  |
| b) | Explain active, reactive and apparent power of a circuit using a power triangle. |  |
| c) | Derive an expression for the power consumed in a R-L circuit. Draw the waveform diagram for the power. |  |
| d) | In the circuit of fig.2, applied voltage is given by (0+j10) volts and the current is (0.8 + j 0.6) amps. Find the value of R and X and also indicate whether X is inductive or capacitive. |  |
| e) | A balanced star connected load of (8 + j 6) ohms per phase is connected to a balanced 3-phase, 400V supply. Find line current and power consumed. |  |
| f) | Derive an expression for the transient current in R-L, D.C. circuit. |  |
| g) | Explain the different types of transients. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the current in each branch for the circuit shown in fig.3 by using nodal method. |  |
| b) | Find the load current I of the circuit shown in fig.4 by using source transformation. |  |
| c) | For the circuit shown in fig.5, determine the value of RL that will cause maximum power transfer in RL. Also determine the power under this condition. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A choke takes a current of 0.5 A when connected across 230V, 50Hz, 1-phase, A.C. supply and consume 40W. Calculate i) p.f. ii) Inductance of the coil & iii) Resistance of the coil. |  |
| b) | A resistance of 20 ohms, inductance of 0.01H and capacitance of 200 microfarads are connected in series across 230V, 50Hz, 1-phase supply. Calculate i) Impedance of the circuit ii) Current through the circuit and iii) power consumed. |  |
| c) | For the circuit shown in fig.6, calculate the current in each branch and total circuit. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Explain the resonant frequency, Q-factor and bandwidth of a series circuit. - Draw and explain the graphic representation of parallel resonant circuit. | (03) (03) |
| b) | An inductor having a resistance of 25 ohms and a Qo of 10 at a resonant frequency of 10 KHz is fed from a 1000 volts supply. Calculate i) Value of series capacitance required to produce resonance with the coil ii)The inductance of the coil - Voltage across capacitance iv)Voltage across the coil. | ………2/- |

-2-

| c) | A coil of resistance 20 ohms and inductance 200 mH is in parallel with a variable capacitor. This combination is in series with a resistor of 8000 ohms. The voltage of the supply is 200V at a frequency of 106 Hz . Calculate i) The value of capacitor to give resonance ii) The Q of the coil & iii) current in each branch of the circuit at resonance. |  |
| --- | --- | --- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a circuit diagram, explain the Hay’s bridge. Give its equation at balance. What are its advantages and disadvantages? |  |
| b) | The arms of an a.c. Maxwell’s bridge are arranged as follows: AB is a non-inductive resistance of 1000 ohms in parallel with a capacitor of capacitance 0.5 F. BC is a non-inductive resistance of 600 ohms, CD is an inductive impedance (unknown) and DA is a non inductive resistance of 400 ohms. If balance is obtained, find the value of resistance and inductance of branch of CD. |  |
| c) | A 3- phase, 4-wire system having a 200V line to neutral has loads connected between the respective lines and neutral: ZR=100, Zy=1036, and ZB= 10-60 ohms. Calculate the line current in the neutral wire and total power taken. Assume phase sequence as RYB. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The line currents in the R,Y and B lines are IR = 300300 A, Iy= 500240 A & IB=100060 A. Calculate the values of the positive, negative and zero phase sequence components. |  |
| b) | A 400V, 3-phase voltage is applied to a balanced delta connected 3- phase load of phase impedance (15+ j 20) ohms. Find the current in each line and power consumed. What is the phasor sum of the three line currents? |  |
| c) | Three identical impedances are connected in star to a 3 phase, 400V, 50 Hz supply. The line current is 35A and the total power taken from supply is 14.5 KW. Calculate the resistance and inductance of each impedance. |  |