## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical & Electronics Engg.

Subject: Power Generation, Transmission & Distribution(5091)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the advantages of high load factor? |  |
| b) | Define the terms diversity factor and demand factor. Give their importance. |  |
| c) | Explain the concept of feeder, distributor and service mains. |  |
| d) | Briefly explain the 4-wire, 3 phase A.C system. |  |
| e) | Give at least three I.E rules for earthing. |  |
| f) | Explain what do you mean by earthing. Why is it necessary? |  |
| g) | What are the advantages and disadvantages of HVDC transmission system? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the important points to be taken into consideration while selecting the size and number of units for a given load curve. |  |
| b) | Draw the schematic diagram of a nuclear power station and discuss its operation. |  |
| c) | Write a short note on Availability and Economical Feasibility of solar power plant. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Why are suspension insulators preferred for high voltage power system transmission? |  |
| b) | Give reasons why A.C.S.R conductors are preferred for transmission and distribution lines. |  |
| c) | Discuss the various types of line supports. |  |
| d) | Define and explain string efficiency. Can its value be equal to 100%? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the advantages and disadvantages of Corona. |  |
| b) | Explain how the electrical breakdown can occur in an insulator. |  |
| c) | With a neat sketch explain pipe earthing system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare ring and interconnected distribution systems on the basis of load, reliability of supply and economy. |  |
| b) | Explain with a neat single line diagram a radial distribution system. What are its advantages and disadvantages? |  |
| c) | Compare D.C. -2 wire A.C. single phase and A.C three phases, 3-wire distribution system with respect to the volume of conductor material required. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare cleat, PVC casing and capping and conduit wiring system w.r.t various factors affecting the selection of wiring system. | ……2/- |

-2-

| b) | Draw the circuit diagram of stair case wiring and Godown wiring and explain. |  |
| --- | --- | --- |
| c)i) | Draw the electric circuit diagram of a wiring consisting of one light point, one fan point and one 5A socket outlet to be installed in a room. |  |
| ii) | Give atleast three I.E rules for domestic wiring system. |  |

***********