## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: Transformers(4106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | For transformer, define - Primary winding - Step up transformer. |  |
| b) | What is leakage flux? Why a transformer should have low leakage flux? |  |
| c) | Define a per unit quantity. State formula for transforming per unit quantity to new KVA base & new KV base. |  |
| d) | Briefly explain how you will ensure that the polarity of the two single phase transformers to be connected in parallel is correct. |  |
| e) | State any two advantages and any one disadvantage of an autotransformer over two winding transformer. |  |
| f) | List special features of Furnace transformer. |  |
| g) | State any three uses of equivalent circuit of transformer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain construction and working of “OFF Load Tap Changer’. |  |
| b) | With neat diagram explain: - Spiral winding - Cross over winding. |  |
| c) | Briefly explain following methods of cooling of transformer: - ONAN - OFWF. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the equivalent circuit of transformer & briefly explain the significance of each parameter. |  |
| b) | A transformer takes 10 ampere on no load at a p.f. of 0.1. The turns ratio is 4:1 (step down). If the secondary supplies a load of 200 amperes at a p.f. of 0.8 lagging, find primary current & its p.f. |  |
| c) | The parameter of a 2300/230 V, 50 Hz transformer are given below. R1=0.286 Ω, R2= 0.319 Ω X1=0.73 Ω, X2 =0.73 Ω Ro=250 Ω, X0=1250 Ω The secondary load impedance ZL =0.387+j0.29Ω. Using approximate equivalent circuit, determine - The primary current - The efficiency. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define efficiency of a transformer. State and prove conditions for maximum efficiency. |  |

………2/-

-2-

| b) | A 50 KVA, 11KV/440V has iron loss of 3 KW and full load copper loss of 5 KW. Determine its efficiency at - 75% of full load at 0.8 pf. - 25% of full load at 0.8 pf. |  |
| --- | --- | --- |
| c) | A 100 KVA,11KV/440V has total resistance referred to secondary is 0.75 ohm and total reactance referred to secondary is 0.9 ohm. Determine percentage voltage regulation when the transformer is supplying full load at a p.f. of (i) 0.8 lag (ii) 0.9 lead. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain connection diagram & vector diagram of the following transformer: (i) Dy 1 (ii) Yy 6. |  |
| b) | Two single phase transformers A & B rated at 250 KVA each operate in parallel. The impedances for A and B are (1+j6) Ω and (1.2 + j 4.8) Ω respectively. Compute the load shared by each when the total load is 500 KVA at 0.8 pf lagging. |  |
| c) | Find the all day efficiency of a 50 KVA distribution transformer having full load efficiency of 94% and full load copper losses are equal to the constant iron losses. The loading of the transformer is as follows, the power factor being 1.0: - Full load for 5 hours - 25% load for 6 hours - 50% load for 3 hours - No load for 10 hours. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 40 KVA, 3300/240 V, 50Hz, 1-phase transformer has 660 turns on primary. Determine - The number of turns on the secondary - The maximum value of flux in the core - The approximate value of full load primary current. |  |
| b) | Write a short note on Scott connected transformer. |  |
| c) | Write a short note on high reactance welding transformer. |  |

***********