## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: A.C. Machines(4113)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Briefly explain how double cage induction motor has better starting performance. |  |
| b) | State advantages and disadvantages of autotransformer starter as compared to star-delta starter. |  |
| c) | Why single-phase induction motor is not self-starting? List different types of single phase induction motors. |  |
| d) | Explain why alternator winding is distributed & short pitched. |  |
| e) | List the methods of synchronising alternators. Explain any one method in brief. |  |
| f) | What is ‘hunting’ of synchronous motor? How it can be reduced? |  |
| g) | Briefly explain working principle of Reluctance motor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive an expression for the torque of induction motor and obtain condition for maximum torque. |  |
| b) | With neat diagram explain construction of three phase squirrel cage induction motor. |  |
| c) | A 14 pole, 50 Hz induction motor runs at 415 rpm. Deduce the frequency of current in the rotor winding & the slip. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a power flow diagram of induction motor. State & briefly explain each loss. Out of these losses which loss is neglected? |  |
| b) | Draw a circle diagram for 3.73KW, 200Volt, 4 pole, 50 Hz, 3-phase, star connected induction motor from following test data i) No load test : 200 V, 5A, 350W ii) Blocked Rotor test : 100 V, 26A, 1700W. For full load condition estimate line current & its power factor. |  |
| c) | The shaft output of a 3-phase induction motor is 75KW. The friction and windage losses are 1KW. The stator core losses are 4 KW and stator copper losses are 2.5 KW. If the slip is 3.5%, what is the efficiency of the motor? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of power circuit diagram and control circuit diagram, explain working of fully automatic star-delta starter. |  |
| b) | With the help of neat diagram, explain construction and working of permanent capacitor type single phase induction motor. |  |
| c) | The stator of a three phase, 8-pole synchronous generator driven at 750 rpm has 72 slots. The winding has been made with 36 coils having 10 turns per coil. Calculate the rms value of induced emf per phase if flux per pole is 0.16 weber. Assume that coil is short pitched by one slot. | ………2/- |

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | State the effect of armature resistance, leakage reactance and synchronous impedance on alternator voltage. Draw the phasor diagram of alternator under no load condition and full load condition for inductive load. |  |
| b) | Explain how synchronous impedance of alternator can be found out using open circuit and short circuit test on it. |  |
| c) | In a 2000 volt, single phase alternator a full load current of 100 amperes is produced on short-circuit by a field excitation of 2.5 amperes; an emf of 500 V is produced on open circuit by same excitation. The armature resistance is 0.8 ohm. Determine the voltage regulation when the generator is delivering 100 amperes at - 0.71 pf lagging - 0.71 pf leading. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain principle of operation and construction of Repulsion motor. |  |
| b) | What are V-curves? How they are determined experimentally? |  |
| c) | Two alternators A and B operate in parallel and supply a load of 8 MW at 0.8 pf lagging. The power output of A is adjusted to 500KW by changing its steam supply ands its pf is adjusted to 0.9 lagging by changing its excitation. Find the pf of alternator B. |  |

***********