## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-II(4061)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain ‘Ring groove fortification’. |  |
| b) | Give the objectives of supercharging. |  |
| c) | List cause of high oil consumption in I.C Engines. |  |
| d) | Explain briefly properties of lubricating oils. |  |
| e) | Discuss on the service ratings of lubricating oils. |  |
| f) | A 4-cylinder, 4-stroke S.I. Engine having Bore x Stroke as 100 mm x 95 mm runs at 5000 r.p.m. Calculate the volume of the air fuel mixture drawn in one hour. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Using a neat sketch explain the High speed circuit of a Solex-Mikoni carburetor. |  |
| b) | Explain the working of a Mechanical Governor with the help of a neat sketch. What is the difference between Governor and Flywheel? |  |
| c) | Discuss the importance of Diesel filtration and explain how it is achieved in a CI Engine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a sketch the L.T. Magneto Ignition system. How does it differ from the H.T. one? |  |
| b) | Discuss the Vacuum Advance control of spark in S.I. Engine. |  |
| c) | Explain the requirements of an efficient Spark Ignition system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are hybrid cars? Discuss with neat sketches any two types. |  |
| b) | Sketch the Hydraulic valve lifter and discuss its working and advantages. |  |
| c) | With a neat line diagram explain the supercharging method using a turbocharger. List its advantages. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how valve overlap and control of vacuum advance affect the pollutants in the exhaust. |  |
| b) | Describe the Air Injection Method to clean the exhaust gas. |  |
| c) | Explain with a sketch ‘crankcase ventilation’ and its importance. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the Air consumption measurement in an IC Engine using the Air Box Method. |  |
| b) | Explain the procedure of calculating the IHP by Electronic Indicator. |  |

……..2/-

|  | -2- |  |
| --- | --- | --- |
| c) | The Morse Test of a petrol Engine provided the following results: B.P. with all cylinders working = 73.6 KW B.P. with No.1 cylinder cut out =50.4 KW B.P. with No.2 cylinder cut out =50.8 KW B.P. with No.3 cylinder cut out =52.3 KW B.P. with No.4 cylinder cut out =52.6 KW. Calculate the F.P., I.P. and Mechanical Efficiency. |  |

***********