## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Computer Engg.

Subject: Computer Organisation-I(4220)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Given x = (00101)2, which equals (+5)10 and y = (001101)2, which equals (+13)10 in 2’s complement notation, compute the difference d = x-y |  |
| b) | Define Bus Cycle. |  |
| c) | Draw a block diagram of a micro programmed control unit. |  |
| d) | State three characteristics of multiprocessors. |  |
| e) | Briefly explain two-address and one-address instructions with an example. |  |
| f) | Let a = (10111)2 and b =(10011)2 in S & M notation. Compute the product p = a x b. |  |
| g) | Explain selective set and selective clear logic micro- operations with an example of each. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With a block diagram and space time diagram briefly explain instruction look ahead pipelining. |  |
| b) | Draw internal structure of a CPU (general structure) and explain. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a diagram for bus system with four registers. Explain in brief and also give function table. |  |
| b) | Draw a 4-bit arithmetic circuit and explain in brief. |  |
| c) | With a diagram explain the construction of a bus system with three state buffers. |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Draw the hardware and flowchart for multiplication of signed 2’s complement numbers by Booths method. |  |
| b) | - Draw the hardware implementation for division operation. - With an example illustrate binary division with digital hardware. ### OR iii) Draw flowchart for divide operation. | (3) (3) (3) |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain synchronous v/s Asynchronous operation. |  |
| b) | Write four points to explain system bus operation. |  |
| c) | Explain stored program organisation with diagrams. |  |
| Q.No.6. | Write short notes: (any three) | 3 x 4 = 12 |
| a) | Relative addressing mode |  |
| b) | Arithmetic shifts |  |
| c) | Tightly coupled multiprocessor organisation |  |
| d) | Loosely coupled multiprocessor organisation |  |

***********