## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Common

Subject: Elements of Civil Engineering(3003)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Enumerate different types of piles. |  |
| b) | Explain how do you eliminate parallax. |  |
| c) | What is meant by grouting? State it uses. |  |
| d) | List down various materials used for thermal insulation. |  |
| e) | What is the purpose of foundation? |  |
| f) | What is meant by safe bearing capacity of soil? State its significance. |  |
| g) | Draw a neat sketch of King Post roof truss. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch & explain procedure you will adopt to overcome obstacles such as river. |  |
| b) | Explain ‘method of stepping’ while chaining on sloping ground. |  |
| c) | - Distinguish between whole circle bearing system & quadrantal bearing system. - If F.B of line is 27, what will be the back backing of that line? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following consecutive readings were taken with level and the levelling staff on continuously sloping ground at common interval of 30m. 0.905 (pt.A), 1.745, 2.345, 3.125, 3.725, 0.545, 1.39, 2.055, 2.955, 3.455, 0.595, 1.015, 1.850, 2.655, 2.945. RL of ‘A’ was 395.5m. Calculate RL’s of different points. |  |
| b) | Explain “Rise & Fall method” of calculating RL with example. State its advantages over the other method. |  |
| c) | Describe method of measuring horizontal angle by using Theodolite. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure, how do you prolong a line using Theodolite. |  |
| b) | List the instrument used in levelling. Explain construction of each. |  |
| c) | Write short note on sound Insulation. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of sketch explain construction and use of grillage foundation. |  |
| b) | List various types of flooring adopted in residential building & explain Terrazo flooring in brief. |  |
| c) | Explain following stages of concreting operations: i)Transportation ii) Compaction. |  |

………2/-

-2-

| Q.No.6. | Using the line plan, shown in Fig.(1), draw to suitable scale to show the following: - Plan - Elevation - Section. Assume suitable data as per rules & regulations. | (12) |
| --- | --- | --- |