## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-I(4054)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define “Compression Ratio” and “Stroke” for a two stroke CI engine. |  |
| b) | Classify I.C Engines. (any 6 types) |  |
| c) | Explain with sketches a “starting and warming” circuit for a complete carburetor. |  |
| d) | With the sketch of a graph, explain the Air-fuel mixture requirement for maximum power and minimum s.f.c |  |
| e) | Define “Octane Number”. |  |
| f) | A radiator has brass water tubes plus aluminium fins. Why? |  |
| g) | Explain the functions of the thermostat. Locate its position in the cooling system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List at least six points to compare between two stroke and 4 stroke engines. |  |
| b) | With sketches describe the various shapes of piston rings and their particular functions. |  |
| c) | Explain the valve timing diagrams of 2-stroke and 4-stroke SI Engines. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List and explain the requirements of an ideal gasoline for use in I.C Engines. |  |
| b) | Using a neat sketch explain the construction, working and drawbacks of AC Mechanical petrol pump. |  |
| c) | For a carburetor what are Steady state operations and Transient state operations. Explain them in detail. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the Air cooling of I.C Engines and state its advantages and disadvantages. |  |
| b) | Nozzles are used on Diesel Injectors. Explain these nozzles and specific use of each, with the help of neat sketches. |  |
| c) | Using sketches describe the Jerk pump and unit Injector systems. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the objectives of engine lubrication? Explain the Mist Lubrication system. |  |
| b) | Explain with sketches at least 4 types of CI Engine combustion chambers. Give their merits and drawbacks. |  |
| c) | Using neat sketches explain the abnormal combustion of detonation in S.I Engines. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Name at least six brands of engine lubricants sold in the market. Briefly explain the requirements of a good engine lubricant. |  |
| b) | A 3 cylinder, 4 stroke I.C Engine having bore =10cm, stroke =12cm gives an average torque of 112 N-m when running at 2100 rpm. Calculate the B.P and b.m.e.p of the engine. |  |
| c) | With a neat sketch explain the working of a Rope Brake dynamometer. |  |

***********