## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics Engineering/EC/MED/E&EE

Subject: Analog Electronics(4140)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Drawn the circuit of a dual input balanced output differential amplifier. List the other configuration of the differential amplifier. |  |
| b) | Explain why open loop opamp is generally not used in linear applications giving examples. |  |
| c) | Draw the circuit of an ideal and practical differentiator circuit. |  |
| d) | Define the terms lock range and capture range of PLL. |  |
| e) | What is an analog computer? Draw four symbols used to represent the blocks in an analog computer. |  |
| f) | State at least 3 advantages of active filters over passive filters. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A non-inverting amplifier with R1 =1 K & R2 = 10K. The various opamp parameters are AOL= 2 x 105, Ri = 2 M, Ro = 75, fo = 5 Hz, + Vsat = 12V. Calculate the closed loop gain, input resistance, output resistances, bandwidth with feedback and total o/p voltage offset. |  |
| b) | Define the following parameters of an opamp: (i) CMRR (ii) Slew rate (iii) PSRR. State their typical and ideal values. |  |
| c) | Explain with neat circuit diagram, the operation of an instrumentation amplifier using opamp. Give necessary equations. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with diagram the operation of a non-inverting summing amplifier using opamp. Give necessary equations. |  |
| b) | Explain the working of Wein-bridge oscillator using opamp. Draw circuit diagram and output waveform. |  |
| c) | Explain the operation of an astable multivibrator using opamp. Draw circuit diagram and waveform. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw circuit diagram and explain the operation of integrator. Draw output waveforms for a sine and triangular input waveforms. |  |
| b) | Explain the operation of a Schmitt trigger circuit using opamp. Draw circuit diagram and necessary waveform. |  |
| c) | Explain the working of a sample and hold circuit. Draw circuit diagram and waveform. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With help of circuit diagram explain operation of a low pass filter. |  |
| b) | Draw and explain the block diagram of a phase lock loop. |  |
| c) | For the PLL circuit RT = 10 KΩ, CT = 0.005μF. Calculate the following: (i) Free running frequency (fout) (ii) Lock range fL (iii) Capture range fC. Plot the relationship between fout, fL and fC. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain why protection circuits are required in voltage regulators. |  |
| b) | Explain the operation of simple current limiting circuit & overvoltage protection circuit. Draw circuit diagram. |  |
| c) | Explain the following terms: - Amplitude scaling - Time scaling. |  |

***********